������������������������������������������������������������������������������

THE P5 BASICS, AND THE NEW PROFILING REGISTERS

By TalkieToaster

������������������������������������������������������������������������������

RELEASE 2 (01/01/96)

DISCLAIMER: I will accept no responsibility whatsoever for any loss or

damage that may occur directly or indirectly by the use or misuse of

this information.

Mail: tim@legend.co.uk

IRC: TimJ

If any technical information is incorrect, PLEASE let me know.

������������������������������������������������������������������������������

CONTENTS

������������������������������������������������������������������������������

1. INTRODUCTION

2. THE P5 PIPELINES

3. THE NEW PROFILING REGISTERS

4. SOME MSR AND TSC MACROS

������������������������������������������������������������������������������

1. INTRODUCTION

������������������������������������������������������������������������������

This time round I've gone into more detail. There's still no ground breaking

information in here, but all the same it's still useful if you want the

basics.

New :

- More about the P5 pipelines

- Some TSC and MSR macros for profiling and timing assembly code

������������������������������������������������������������������������������

1. THE P5 PIPELINES

������������������������������������������������������������������������������

First of all... writing about the pipeline is REAL boring.. I hate it.

If you want detailed information about the dual pipelines I suggest you

either find another good p5 doc, or muck about with it.

Right, the P5 has two instruction pipelines - U and V. The V pipe can only

execute selected RISC instructions which are generally ones that take 1

cycle.

If it is possible, the V-pipe will execute the next available instruction at

the same time as the U-pipe. To visualise this:

instructions	  cycles no.	pipe

mov eax,1	; 1   		executed in U-pipe

mov edx,2	; 1   		executed in V-pipe

mov ecx,3	; 2   		executed in U-pipe

mov ebx,4	; 2    		executed in V-pipe

Total 2 cycles.

The V-pipe isn't a full pipeline and can only execute these instructions:

MOV   AND

OR    XOR

ADD   SUB

CMP

For the above instruction - if using an address (memory) as the destination,

it can only be execute if there is NO displacement.

eg: mov [mydata + 4],22 will NOT execute in the V-pipe.

INC   DEC

PUSH  POP

TEST  LEA

JMP   CALL

JCC

FXCH - This is the only fpu instruction that will execute in the V-pipe.

The P5 can only execute these in the V-pipe is one of these instructions are

in the U-pipe. These are called "pairable" instructions.

MOV   AND

OR    XOR

ADD   SUB

ADC   SBB

INC   DEC

CMP   TEST

PUSH  POP

LEA   SHL

SHR   SAL

ROL   ROR

RCL   RCR

FXCH - i think :)

The same applies to the MOV,AND,OR etc.. instructions with displacements. If

they contain a displacement the V-pipe stalls.

You can check all the stalls using the profiling registers, or the TSC (see

the later section on these).

In general, pairable instructions are those RISC-type ones.

If the V-pipe can't execute an instruction, it "stalls" and passes it to the

U-pipe on the next cycle. So the V-pipe skips an instruction.

If the instruction in V pipe depends on the instruction in the U pipe then

the V pipe stalls and misses a cycle.. eg:

add eax,ecx	; U-pipe.. 1 cycle

add eax,ecx	; can't execute in V because eax isn't know until

; above instruction is comeplete.. so execute in

; U-pipe.. 1 cycle

Total 2 cycles

On the other hand

add eax,ecx	; U-pipe.. 1 cycle

add ebx,ecx	; V-pipe.. same cycle

Total 1 cycle , because they both execute at once.

Now, onto AGI's. Address Generation Interlocks, are evil. On the 486 AGI's

occured if you used a register 1 cycle before using it in an EA (Effective

Address). This stalled the addressing pipeline and gave you a 1 cycle penalty

while it recalulated the EA.

All you need do on the pentium is make sure there is 1 cycle between the

register load and using it in an EA. This includes both pipes.. if the

instructions pair, then you may not be leaving 1 cycle between the two.

If the instruction going through a pipe require memory read/writes

and is NOT mov,push or pop, then it cause a lockstep.

This means the other pipe is locked for all until the last last cycle:

instructions	  cycles no.	pipe

add [edx],eax	; 1     	U-pipe, causes lockstep..

; 2

dec ecx		; 3		V-pipe, can't execute until

;		last cycle of the add.

Another thing... uncached code causes a pipeline stall. The code needs to

be executed at least twice before it is DEFINETLY in the cache. On the

2nd loop some code may still not be cached. I did some tests and usually

it cached after the first loop, but other times it wasn't. I suppose it

depends on the state of the code cache at that point.

Pahaps it was a freak of the code I was using.. do some tests and see what

you come up with.

������������������������������������������������������������������������������

3. THE NEW PROFILING REGISTERS

������������������������������������������������������������������������������

The P5 also has some internal profiling and timing registers that can be

accessed through RDTSC, RDMSR and WRMSR. These allow you to count cycles and

events like cache misses etc..

RDTSC - Read Time Stamp Count

The TSC is incremented every cpu cycle. We can use it to get a

cycle count.

RDMSR - Read an MSR register. MSR index in ECX, value put in EDX:EAX.

WRMSR - Write an MSR register. MSR index in ECX, value to store in EDX:EAX.

If you don't have a p5 compiler these are the opcodes:

RDTSC - db 0fh, 031h

RDMSR - db 0fh, 032h

WRMSR - db 0fh, 030h

The values are all 64 bit (EDX:EAX). All the counters are reset on power-on.

So the TSC variable can last about 5800 years on a P100 if you left it on

all the time.

Your code need to be running at CPL0 to execute RDMSR or WRMSR. So if you are

running Window, or in a Win95 DOS BOX, then it will crash.

Also the TSC counts ALL cycles. So the cycle count includes those of any

multitasking environment you're in (like Win95).

The MSR registers are pretty cool. Or more to the point, registers 11h,12h

and 13h are cool.

You see, register 11h can be used to tell two timers what to count. These

timers can be read from 12h and 13h. Using these timers you can count lots

of profiling information like data reads, data read misses, pipe stalls

and misaligned data references etc. etc.

Armed with these timers and the TSC, you can tell exactly what you're code

is doing and where cycles are lost.

To tell the timers what to count you set the bits in 11h like so:

EDX:EAX - 64 bit value

bit 0  - 15 = Timer #0

bit 16 - 31 = Timer #1

bit 32 - 63 = Reserved

The bits for each timer are set like so:

bit 0  - 5  = Event to count (list of events follows)

bit 6       = Enable count in CPL 0,1 and 2 (system code)

bit 7       = Enable count in CPL 3 (user code)

bit 8       = 1 = Count cycles, 0 = Count events

bit 9       = PM0 selection. 1=pin shows overflows, 0=pin shows increments.

bit 10 - 15 = Reserved

Generally you can leave most of the bits alone. Set bits 0-5 to the event

type, and set either bit 6 or 7. Under most dos-exteders your code runs at

CPL0 (bit 6), but it the counting doesn't seem to work, try bit 7 instead.

So, to count an event you'd do something like this:

; Set timers...

mov ecx,011h

mov eax,TIMER#1 bits...

shl eax,16

mov eax,TIMER#0 bits...

WRMSR

; Read start count

mov ecx,012h

RDMSR

mov _timer0,eax		; just save bottom 32 bits.

mov ecx,013h

RDMSR

mov _timer1,eax

;-----

;Do your code here

;-----

; Read end count

mov ecx,012h

RDMSR

mov sub eax,_timer0	; subract start count

mov _timer0,eax		; save value

mov ecx,013h

RDMSR

sub eax,_timer1

mov _timer1,eax

And that's it. Of course you have to take into account the overhead code

for timing whatever event you want.

Event types:

00h - Data reads

01h - Data writes

02h - Data TLB misses

03h - Data read misses

04h - Data write misses

05h - Writes (hits) to M or E state lines

06h - Data cache lines written back

07h - External snoops

08h - Data cache snoop hits

09h - Memory accesses in both pipes

0Ah - Bank conflicts

0Bh - Misaligned data memory references

0Ch - Code reads

0Dh - Code TLB misses

0Eh - Code cache misses

0Fh - Any segment register loaded

10h - Segment descriptor cache accesses

11h - Segment descriptor cache hits

12h - Branches

13h - Branch Target Buffer hits

14h - Taken branches or BTB hits

15h - Pipeline flushes

16h - Instructions executed in both pipes (incl. fpu instructions I think)

17h - Instructions executed in the v-pipe

18h - Clocks while bus cycle in progress (bus utilization)

19h - Pipe stalled by full write buffers (writes backup)   - cycles lost

1Ah - Pipe stalled by waiting for data memory reads        - cycles lost

1Bh - Pipe stalled by writes to M or E lines               - cycles lost

1Ch - Locked bus cycles

1Dh - I/O read or write cycles

1Eh - Non-cacheable memory references

1Fh - Pipeline stalled by address generation interlock (AGI) - cycles lost

20h - unknown, but counts

21h - unknown, but counts

22h - Floating-point operations

23h - Breakpoint matches on DR0 register

24h - Breakpoint matches on DR1 register

25h - Breakpoint matches on DR2 register

26h - Breakpoint matches on DR3 register

27h - Hardware interrupts

28h - Data reads or data writes

29h - Data read misses or data write misses

������������������������������������������������������������������������������

4. SOME MSR AND TSC MACROS

������������������������������������������������������������������������������

(See also the example files and header files.)

These are basically self explanitary I think, mail me if any problems.

Oh, they we're tested in 32bit protected mode.

******** SECTION 1 - TSC CYCLE COUNTER **********

These Macros require a memory variable, _TSC , a dword.

This varaible is used to store the count takes. In StartTSC, it is cached

before the start value is stored, so we won't get an unpredictable cache

miss.

StartTSC:

- cache the stack memory

- cache _TSC count variable

- save registers

- stall the pipeline with an idiv.

- get TSC register (this is definetly executed in the U-pipe)

- save the TSC count in the cached _TSC memory variable.

- pop registers

EndTSC:

- push registers

- stall pipeline with idiv (maybe not needed, but just to be sure)

- get the TSC register

- subtract overhead cycles

- subtract start value

- save count in _TSC memory variable.

It may look strange the way I've stalled the pipeline with an idiv. But the

RDTSC instruction seems to pair with almost any other instruction. Note. two

non-pairable instructions in a row would have worked as well (like cdq cdq).

The only problem I can see is with the push at the start of EndTSC. The

overhead cycles I calculated assume a cache hit on the push. If you get a

cache miss then the TSC count will be too high. This should only occur when

you time LOTS of code that completely pushes the cached stack out of the

cache. I hope this will be a very rare occasion. If anyone can suggest a fix

for this I'd be happy to put it in.

;�����������������������������������������������������������������������������

; Macros for the TSC cycle counter

; To get 'propper' cycle counts from the TSC.. The other timing macros

; are event based, these TSC macros hopefully get true cycle counts from

; code.

;

; There may be a better way to do this, but this works..

; The imul completly stalls the pipeline :) and makes sure the instruction

; pairing is predictable...

;

; Try using these macros after a CALL and after some code and see how

; changing/removing the imul affects things.

;

;�����������������������������������������������������������������������������

; Start TSC cycle counter..

;

StartTSC	MACRO

; Try removing the following pushad/popad, then using

; StartTSC/EndTSC just after a CALL.. Any you'll see the

; stack cache-miss penalty..

;

pushad		; cache stack stuff (for popd after RDTSC)

popad

pushad

mov eax,_TSC		; cache _TSC

imul _TSC		; stall pipeline. RDTSC executes in U-pipe...

;cdq			; these also work.

;cdq

db 0fh,031h		; RDTSC - get start count

mov _TSC,eax		; save start count

popad

ENDM

; End TSC cycle counter..

;

EndTSC		MACRO

; Note.. the pushad is affected by the cache.

; no way to get around this one :(

; Just remeber this, if you're code pushes cached stack values out of

; the cache then you'll have an inacurracy.

pushad		; does not pair.. so stalls pipeline..

imul edx		; this may not be needed, but it's just incase

; the pushad decides to pair with RDTSC..

; It's late and I want to write somethinng else :)

db 0fh,031h		; RDTSC - get end count

sub eax,27 		; overhead cycles..

sub eax,_TSC		; get range of count

mov _TSC,eax		; save range

popad

ENDM

; Just for quick referencing.

s	equ StartTSC		; Start time

e	equ EndTSC		; End time

******** SECTION 2 - MSR TIMING MACROS **********

These macros will program and read the MSR timers. StartProfile will time

the overhead for the particular event, and subtract this at the end.

For getting cycle counts this is completely useless, use StartTSC and EndTSC

for that.

If the timing seems wrong, change the CPL at which these work. Under a

normal system CPL0..2 is where you're code runs. But under other systems

or windows, there's a good change it will run at CPL3.

To fix this, in the SetMSRTimers macro, make it AND the event with USER_CODE

(CPL3 code) instead of SYS_CODE (CPL0..2 code).

The way it times the overhead isn't perfect, I know. I tested this with some

of the events, and it hasn't failed yet. Subtracting the overhead gets better

counts and allows both timers to get the same count on a particular event.

If anyone finds any particular event where these macros go completely wrong

then let me know.

alterations I should make in the near future:

- stall the pipeline as in StartTSC/EndTSC

- cache the memory values at the start

Other than that these macros are pretty good.

They need several dword variables, these are:

_prof0		dd	0		; Timer0 profile count

_prof1		dd	0		; Timer1 profile count

_profsub0	dd	0		; Overhead count for timer0

_profsub1	dd      0		; Overhead count for timer1

_profdsub0	dd	0		; dummy sub value for timing

_profdsub1	dd      0		; the overhead.. must stay 0

Since we sub the overhead at the end, when timing the overhead code we need

to do a dummy sub.

Mail me with any problems.

;�����������������������������������������������������������������������������

; General MSR timing macros

;�����������������������������������������������������������������������������

; Timer bits:

;  0-5  - Event

;  6    - Count system overhead (CPL 0-2)

;  7    - Count user code (CPL 3)

;  8    - 0 = Count events, 1 = count cycles

;  9    - 0 = PM0 slection, show incs, 1 = show overflows

USER_CODE    = 0010000000b

SYS_CODE     = 0001000000b

COUNT_EVENTS = 0000000000b

COUNT_CYCLES = 0100000000b

PM0_INCS     = 0000000000b

PM0_OVERFLOWS= 1000000000b

; Setup the MSR timers to count something.

;

; Trashes eax ecx edx

;

SetMSRTimers	MACRO	TIMER0,TIMER1

mov ecx,011h		; MSR 11h

xor edx,edx		; top 32bits empty

mov ax,TIMER1		; timer#0

or  ax,SYS_CODE	; time system code (CPL 0)

shl eax,16

mov ax,TIMER0		; timer#1

or  ax,SYS_CODE	; time system code (CPL 0)

db 0fh,030h		; WRMSR

ENDM

; These just trash ecx...

ReadMSRTimer0	MACRO

mov ecx,012h		; Timer #0

db 0fh,032h		; RDMSR

ENDM

ReadMSRTimer1	MACRO

mov ecx,013h		; Timer #1

db 0fh,032h		; RSMSR

ENDM

;�����������������������������������������������������������������������������

; Macros to profile some code..

;

;�����������������������������������������������������������������������������

StartProfile MACRO TIMER0 , TIMER1

; first get overhead for each timer..

;

; this is by no means perfect, but does allow both timers

; to produce the same results. Without this the timers

; would produce different results when timeing the same

; thing (because timer1 also times the read of timer0)

;

pushad

SetMSRTimers TIMER0 , TIMER1	; setup timers

ReadMSRTimer0

mov _profsub0,eax		; save start of timer0

ReadMSRTimer1

mov _profsub1,eax		; save start of timer1

popad				; get back original regs

; timed code here.. none just timing overhead

pushad

ReadMSRTimer0

sub eax,_profdsub0		; dummy sub

sub eax,_profsub0		; sub start value

mov _profsub0,eax		; save overhead of timer0

ReadMSRTimer1

sub eax,_profdsub1		; dummy sub

sub eax,_profsub1		; sub start value

mov _profsub1,eax		; save overhead of timer1

popad				; get back original regs

; now actually start the timings for real..

;

pushad

SetMSRTimers TIMER0 , TIMER1

ReadMSRTimer0

mov _prof0,eax

ReadMSRTimer1

mov _prof1,eax

popad				; get back original regs

ENDM

EndProfile MACRO

pushad

ReadMSRTimer0

sub eax,_profsub0		; sub overhead time

sub eax,_prof0			; sub start time

mov _prof0,eax			; save timer0

ReadMSRTimer1

sub eax,_profsub1		; sub overhead time

sub eax,_prof1			; sub start time

mov _prof1,eax			; save timer1

popad				; finally restore the original regs

ENDM