******************************************************************************

*****************  DOS/32 Advanced Programmer's Reference ********************

******************************************************************************

Created on:	Feb-03-1997 17:28:04

Updated on:	Feb-27-1997 19:17:10

Copyright (C) Supernar Systems Intl, 1990-1997.

Contents:

---------

1 - DOS/32 Advanced - Introduction

2 - Supported DPMI functions (available through INT 31h)

2.0 - DPMI functions - Overview

2.00 - DPMI function 0000h - Allocate Descriptors

2.01 - DPMI function 0001h - Free Descriptor

2.02 - DPMI function 0002h - Map Segment to Descriptor

2.03 - DPMI function 0003h - Get Selector Increment Value

2.04 - DPMI function 0006h - Get Segment Base Address

2.05 - DPMI function 0007h - Set Segment Base Address

2.06 - DPMI function 0008h - Set Segment Limit

2.07 - DPMI function 0009h - Set Descriptor Access Rights

2.08 - DPMI function 000Ah - Create Alias Descriptor

2.09 - DPMI function 000Bh - Get Descriptor

2.10 - DPMI function 000Ch - Set Descriptor

2.11 - DPMI function 000Eh - Get Multiple Descriptors

2.12 - DPMI function 000Fh - Set Multiple Descriptors

2.13 - DPMI function 0100h - Allocate DOS Memory Block

2.14 - DPMI function 0101h - Deallocate DOS Memory Block

2.15 - DPMI function 0102h - Resize DOS Memory Block

2.16 - DPMI function 0200h - Get Real Mode Interrupt Vector

2.17 - DPMI function 0201h - Set Real Mode Interrupt Vector

2.18 - DPMI function 0202h - Get Processor Exception Handler Vector

2.19 - DPMI function 0203h - Set Processor Exception Handler Vector

2.20 - DPMI function 0204h - Get Protected Mode Interrupt Vector

2.21 - DPMI function 0205h - Set Protected Mode Interrupt Vector

2.22 - DPMI function 0300h - Simulate Real Mode Interrupt

2.23 - DPMI function 0301h - Call Real Mode Procedure with RETF Frame

2.24 - DPMI function 0302h - Call Real Mode Procedure with IRET Frame

2.25 - DPMI function 0303h - Allocate Real Mode Callback Address

2.26 - DPMI function 0304h - Free Real Mode Callback Address

2.27 - DPMI function 0305h - Get State Save/Restore Addresses

2.28 - DPMI function 0306h - Get Raw Mode Switch Addresses

2.29 - DPMI function 0400h - Get DPMI Version

2.30 - DPMI function 0500h - Get Free Memory Information

2.31 - DPMI function 0501h - Allocate Memory Block

2.32 - DPMI function 0502h - Free Memory Block

2.33 - DPMI function 0503h - Resize Memory Block

2.34 - DPMI function 050Ah - Get Memory Block Size and Base

2.35 - DPMI function 0600h - Lock Linear Region

2.36 - DPMI function 0601h - Unlock Linear Region

2.37 - DPMI function 0604h - Get Page Size

2.38 - DPMI function 0702h - Mark Page as Demand Paging Candidate

2.39 - DPMI function 0703h - Discard Page Contents

2.40 - DPMI function 0800h - Physical Address Mapping

2.41 - DPMI function 0801h - Free Physical Address Mapping

2.42 - DPMI function 0900h - Get and Disable Virtual Interrupt State

2.43 - DPMI function 0901h - Get and Enable Virtual Interrupt State

2.44 - DPMI function 0902h - Get Virtual Interrupt State

2.45 - DPMI function 0A00h - Get Vendor-Specific API Entry Point

2.46 - DPMI function 0E00h - Get Coprocessor Status

2.47 - DPMI function 0E01h - Set Coprocessor Emulation

2.48 - DPMI function 0EEFFh - Get DOS Extender Info (PMODE/W compatible)

2.49 - Vendor Specific DPMI API Extensions

2.50 - API function 00h - Get Access to GDT and IDT

2.51 - API function 01h - Get Access to Page Tables

2.52 - API function 02h - Get Access to Internal Interrupt Buffers

2.53 - API function 03h - Get Access to Extended Memory Blocks

2.54 - API function 04h - Get Access to Real Mode Virtual Stacks

2.55 - API function 05h - Get Access to Protected Mode Virtual Stacks

2.60 - DPMI error codes

3 - Supported Extended DOS functions (available through INT 21h)

3.0 - Extended DOS functions - Overview

3.01 - DOS function 09h - Write String

3.02 - DOS function 0Ah - Buffered Input

3.03 - DOS function 1Ah - Set Disk Transfer Area Address (DTA)

3.04 - DOS function 1Bh - Get Default Drive Information

3.05 - DOS function 1Ch - Get Specific Drive Information

3.06 - DOS function 1Fh - Get Default Drive Parameter Block (DPB)

3.07 - DOS function 25h - Set Interrupt Vector

3.08 - DOS function 2Fh - Get Disk Transfer Area Address (DTA)

3.09 - DOS function 32h - Get Specific Drive Parameter Block (DPB)

3.10 - DOS function 34h - Get InDOS Flag Address

3.11 - DOS function 35h - Get Interrupt Vector

3.12 - DOS function 39h - Create Directory

3.13 - DOS function 3Ah - Remove Directory

3.14 - DOS function 3Bh - Change Directory

3.15 - DOS function 3Ch - Create File

3.16 - DOS function 3Dh - Open File

3.17 - DOS function 3Fh - Read from File

3.18 - DOS function 40h - Write to File

3.19 - DOS function 41h - Delete File

3.20 - DOS function 42h - Set Current File Position

3.21 - DOS function 43h - Change File Attributes

3.22 - DOS function 47h - Get Directory Path Name

3.23 - DOS function 48h - Allocate DOS Memory Block

3.24 - DOS function 49h - Deallocate DOS Memory Block

3.25 - DOS function 4Ah - Resize DOS Memory Block

3.26 - DOS function 4Bh - Execute Program (sub-functoin 00h)

3.27 - DOS function 4Ch - Terminate Program

3.28 - DOS function 4Eh - Find First File

3.29 - DOS function 4Fh - Find Next File

3.30 - DOS function 51h - Get PSP Segment

3.31 - DOS function 56h - Rename File

3.32 - DOS function 5Ah - Create Temporary File

3.33 - DOS function 5Bh - Create New File

3.34 - DOS function 62h - Get PSP Selector

3.35 - DOS function 0FF00h - DOS/4G Identification Call

3.36 - DOS function 0FF88h - DOS/32 Advanced Identification Call

4 - Supported Extended VGA/VESA BIOS functions (available through INT 10h)

4.0 - Extended VGA/VESA BIOS functions - Overview

4.01 - VGA BIOS function 1Bh - Read Functionality Information

4.02 - VGA BIOS function 1Ch - Save/Restore VGA State

4.03 - VESA BIOS function 4F00h - Get SuperVGA Information

4.04 - VESA BIOS function 4F01h - Get SuperVGA Mode Information

4.05 - VESA BIOS function 4F04h - Save/Restore SuperVGA State

5 - Supported Extended Mouse functions (available through INT 33h)

5.0 - Extended Mouse functions - Overview

5.01 - Mouse function 0009h - Define Graphics Cursor

5.02 - Mouse function 000Ch - Define Interrupt Subroutine Parameters

5.03 - Mouse function 0014h - Exchange Interupt Subroutines

5.04 - Mouse function 0016h - Save Driver State

5.05 - Mouse function 0017h - Restore Driver State

5.06 - Mouse function 0018h - Set Alternate Mouse User Handler

5.07 - Mouse function 0019h - Return User Alternate Interrupt Handler

5.08 - Mouse function 0020h - Enable Mouse Driver

1.0 - DOS/32 Advanced - Introduction

**************************************

DOS/32 Advanced is a powerful tool for protected mode programmers to

create their 32bit protected mode applications. DOS/32 Advanced has been

designed to be the fastest, most flexible and very compatible with other

software DOS Extender. And it does the job very well indeed.

DOS/32 Advanced is fully compatible with WATCOM C/C++ and with DOS/4GW

DOS Extender which is provided along with WATCOM's compilators. It supports

Clean (also known as "Raw" or "INT 15h"), XMS, VCPI and DPMI system software

that are the standards today and are widely used in different memory managers.

It has a build-in DPMI 0.9 that can be configured by an external setup

program that is provided in DOS/32 Advanced package. It extends most of DOS

functions (INT 21h) that are used in WATCOM's C/C++ libraries. Further more

it extends many of Mouse functions (INT 33h) and some of VESA VBE functions

(INT 10h) that are widely used by protected mode programmers in games and

applications that use SuperVGA video modes.

Thanks to its design, DOS/32 Advanced can be configured to take up

as little as 30KB of conventional (DOS) memory, and require no extended

memory at all. Your protected mode application can be loaded into DOS memory,

into Extended memory, or in both at the same time. Four different schemes

on how the objects (parts of your protected mode program) are loaded into

memory supported. By default, DOS/32 Advanced will load all the objects

(both 16- and 32bit) into Extended memory to emulate DOS/4GW DOS Extender

which does so. However it is possible to configure DOS/32 Advanced to separate

16- and 32bit objects, and load 16bit objects directly into DOS memory where

they can be reached by the real mode DOS, while 32bit objects can still be

loaded into the Extended memory.

Though it may look like DOS/32 Advanced DOS Extender has been written in

in C/C++ or another high level programming language, it is not. It has been

written entirely in assembly language, and that is why it is working so fast,

supports and extends so many functions and is so small in size. Further more,

after initialization of the protected mode DOS/32 Advanced will discard some

of its parts that are no longer needed, additionally freeing some of

conventional memory.

2.0 - DPMI functions - Overview

*********************************

DOS/32 Advanced has a build-in DOS Protected Mode Interface which provides

your protected mode program with nearly full subset of DPMI 0.9 functions.

The "DPMI" is a standard, which allows protected mode programs to get an

access to extended memory, switch into or out from protected mode and invoke

real mode software. DPMI API is accessed through a software interrupt INT 31h

from protected mode. The DPMI function number is placed in register AX. On

successful return, registers will contain values/information returned by the

DPMI function and the carry flag will be cleared. If the function is not

supported by a DPMI host, or DPMI host could not perform the action desired

by the function, it will return with carry flag set and an error code placed

in AX. Note that some implementations of DPMI do not return error codes, but

only set carry flag.

When DOS/32 Advanced is running under Clean (INT 15h), XMS, VCPI, it will

always install and use the build-in DPMI host which is fully compatible with

DPMI 0.9 subset. The major difference between a standard DPMI 0.9 and DOS/32

Advanced DPMI, is that the DOS/32A DPMI will always return error codes in

register AX. However, when running under an external DPMI host (Windows, OS/2

and such, all have a build-in DPMI driver), DOS/32 Advanced will, depending

on how it was configured, either try to detect a VCPI server, and if present

initialize and use it (in this case build-in DPMI will be installed), or use

the present DPMI host that was installed by previously run software. In this

case it is up to the present DPMI host to provide your program with all the

functions the last may need and return error codes. Note that some of DPMI

implementations are quite buggy, some of them will run your protected mode

code at CPL 0, and some at CPL 3, some will execute real mode calls in Real

mode and some in Virtual 8086 mode. Due to all this complications, DOS/32

Advanced will never interfere with the present DPMI host, and the DPMI host

will be responsible for the exception handling, memory allocation and so on.

The following information about DOS/32 Advanced DPMI will show its major

capabilities:

+ descriptors reside in the GDT.

+ protected mode code runs at CPL 0.

+ real mode INTs/IRQs/callbacks executed in actual Real mode under

Clean (INT 15h) and XMS systems, and in Virtual 8086 mode under VCPI.

+ modifiable number of selectors/callbacks/real/protected mode stacks

that will be allocated at startup.

+ user configurable order of detection of VCPI/DPMI servers.

+ support for exception handling which can be turned off.

+ support for debugging through INT 1 and INT 3 software interrupts.

+ support for trapping software INTs in range INT 08-0Fh issued to emulate

hardware IRQs 0-7 when not remapped (can be disabled).

+ automatically callbacks IRQs 0-7 and 8-15 from real mode to protected

mode when installing a protected mode handler for a particular IRQ

+ support for auto-checking of integrity of extended memory blocks when

allocated through DPMI functions 05xxh (can be disabled).

+ support for vendor-specific API extensions (entry point can be obtained

from DPMI function 0A00h).

More detailed information about DOS/32 Advanced DPMI kernel and its

configuration can be found in document "DOS/32 Advanced Technical Reference".

2.00 - DPMI function 0000h - Allocate Descriptors

===================================================

Allocates one or more descriptors in the client's descriptor table. The

descriptor(s) allocated must be initialized by the application with other

function calls.

In:	AX = 0000h

CX = number of descriptors to allocate

Out:	if successful:

CF clear

AX = base selector

if failed:

CF set

AX = error code

8011h - descriptor unavailable

Notes:	a) If more than one descriptor was requested, the function

returns a base selector referencing the first of a

contiguous array of descriptors. The selector values for

subsequent descriptors in the array can be calculated by

adding the value returned by DPMI function 0003h.

b) The allocated descriptor(s) will be set to "data" with

the present bit set and a base and limit of zero. The

privilege level of the descriptor(s) will match the

application's code segment privilege level.

2.01 - DPMI function 0001h - Free Descriptor

==============================================

Frees a descriptor.

In:	AX = 0001h

BX = selector for the descriptor to free

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8022h - invalid selector

Notes:	a) Each descriptor allocated with DPMI function 0000h

must be freed individually with this function, even if it

was previously allocated as part of a contiguous array of

descriptors.

e) DOS/32 Advanced DPMI and DPMI 1.0 hosts will zero any

segment registers which contain the selector specified in

register BX.

2.02 - DPMI function 0002h Segment to Descriptor

==================================================

Maps a real mode segment (paragraph) address onto a descriptor that can be

used by a protected mode program to access the same memory.

In:	AX = 0002h

BX = real mode segment address

Out:	if successful:

CF clear

AX = selector for real mode segment

if failed:

CF set

AX = error code

8011h - descriptor unavailable

Notes:	a) The descriptor's limit will be set to 64 KB.

b) Multiple calls to this function with the same segment

address will return the same selector.

c) The intent of this function is to provide clients with

easy access to commonly used real mode segments such as

the BIOS data area at segment 0040h and the video refresh

buffers at segments 0A000h, 0B000h, and 0B800h. Clients

should not call this function to obtain descriptors to

private data areas.

d) Descriptors created by this function can never be

modified or freed. For this reason, the function should

be used sparingly. Clients which need to examine various

real mode addresses using the same selector should

allocate a descriptor with DPMI function 0000h and

change the base address in the descriptor as necessary,

using the Set Segment Base Address function DPMI function

0007h.

e) DOS/32 Advanced DPMI will automatically allocate data

selector 0040h to map the BIOS data area at segment 0040h.

2.03 - DPMI function 0003h Get Selector Increment Value

=========================================================

The DPMI function Allocate Descriptors (0000h) can allocate an array of

contiguous descriptors, but only return a selector for the first descriptor.

The value returned by this function can be used to calculate the selectors for

subsequent descriptors in the array.

In:	AX = 0003h

Out:	CF clear

AX = selector increment value

Notes:	a) The increment value is always a power of two.

2.04 - DPMI function 0006h - Get Segment Base Address

=======================================================

Returns the 32-bit linear base address from the descriptor table for the

specified segment.

In:	AX = 0006h

BX = selector

Out:	if successful:

CF clear

CX:DX  = 32-bit linear base address of segment

if failed:

CF set

AX = error code

8022h - invalid selector

Notes:	a) Client programs must use the LSL instruction to query the

limit for a descriptor.  Note that on 80386 machines, the

client must use the 32-bit form of LSL if the segment

size is greater than 64 KB.

2.05 - DPMI function 0007h - Set Segment Base Address

=======================================================

Sets the 32-bit linear base address field in the descriptor table for the

specified segment.

In:	AX = 0007h

BX = selector

CX:DX = 32-bit linear base address of segment

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8022h - invalid selector

8025h - invalid linear address (changing the base

would cause the descriptor to reference a

linear address range outside that allowed

for DPMI clients)

Notes:	a) DOS/32 Advanced DPMI and DPMI 1.0 hosts will automatically

reload any segment register which contains the selector specified

in register BX.

2.06 - DPMI function 0008h - Set Segment Limit

================================================

Sets the limit field in the descriptor table for the specified segment.

In:	AX = 0008h

BX = selector

CX:DX  = 32-bit segment limit

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8021h - invalid value (the limit is greater than 1 MB

but the low 12 bits are not set)

8022h - invalid selector

8025h - invalid linear address (changing the

limit would cause the descriptor to

reference a linear address range outside

that allowed for DPMI clients.)

Notes:	a) The value supplied to the function in CX:DX is the byte

length of the segment-1 (ie, the value returned by the

LSL instruction).

b) Segment limits greater than or equal to 1 MB must be

page-aligned.  That is, limits greater than 1 MB must

have the low 12 bits set.

c) This function has an implicit effect on  the "G"

(granularity) bit in an 80386+ descriptor's extended

access rights/type byte; ie, it is the host's

responsibility to set the "G" bit correctly.

d) Client programs must use the LSL instruction to query the

limit for a descriptor.  Note that on 80386 machines, the

client must use the 32-bit form of LSL if the segment

size is greater than 64 KB.

e) DOS/32 Advanced DPMI and DPMI 1.0 hosts will reload any

segment registers which contain the selector specified in

register BX.

f) DOS/32 Advanced DPMI will perform an optional check for

the segment limits greater than 1 MB to have the low 12 bits

set. That is, when the check is disabled, the DOS/32 Advanced

DPMI will not require segment limits to be page-aligned.

This is done to make DOS/32 Advanced more compatible with

DOS/4G(W) DPMI.

2.07 - DPMI function 0009h - Set Descriptor Access Rights

===========================================================

Modifies the access rights and type fields in the descriptor table for the

specified segment.

In:	AX = 0009h

BX = selector

CL = access rights/type byte

CH = extended access rights/type byte

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8021h - invalid value (access rights/type bytes invalid)

8022h   invalid selector

8025h   invalid linear address (changing the

access rights/type bytes would cause the

descriptor to reference a linear address

range outside that allowed for DPMI clients.)

Notes:	a) The access rights/type byte passed to the function

in CL register has the following format:

+---+---+---+---+---+---+---+---+

| P |  DPL  | 1 |C/D|E/C|W/R| A |

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +-> 0=not accessed, 1=accessed

|   +-+-+   |   |   |   +-----> data: 0=read, 1=read/write

|     |     |   |   |           code: must be 1 (readable)

|     |     |   |   +---------> data: 0=expand-up, 1=expand-down

|     |     |   |               code: must be 0 (non-conforming)

|     |     |   +-------------> 0=data, 1=code

|     |     +-----------------> must be 1

|     +-----------------------> must equal caller's CPL

+-----------------------------> 0=absent, 1=present

The DPMI host interprets the value passed to the function

in CH register as follows:

+---+---+---+---+---+---+---+---+

| G |B/D| 0 |Avl|    Reserved   |

+---+---+---+---+---+---+---+---+

|   |   |   |         |

|   |   |   |         +--> ignored

|   |   |   +------------> can be 0 or 1

|   |   +----------------> must be 0

|   +--------------------> 0=default 16-bit, 1=default 32-bit

+------------------------> 0=byte granular, 1=page granular

b) DOS/32 Advanced DPMI and DPMI 1.0 hosts will reload any

segment registers which contain the selector specified in

register BX.

c) Client programs should use the LAR instruction to examine

the access rights of a descriptor.

2.08 - DPMI function 000Ah - Create Alias Descriptor

======================================================

Creates a new data descriptor that has the same base and limit as the

specified descriptor.

In:	AX = 000Ah

BX = selector

Out:	if successful:

CF clear

AX = data selector (alias)

if failed:

CF set

AX = error code

8011h - descriptor unavailable

8022h - invalid selector

Notes:	a) The selector supplied to the function may be either a

data selector or an executable selector. The created alias

selector will always be a writeable data selector.

b) The descriptor alias returned by this function will not

track changes to the original descriptor.  In other

words, if an alias is created with this function, and the

base or limit of the original segment is then changed,

the two descriptors will no longer map the same memory.

2.09 - DPMI function 000Bh - Get Descriptor

=============================================

Copies the descriptor table entry for the specified selector into an

8-byte buffer.

In:	AX = 000Bh

BX = selector

ES:EDI = selector:offset of 8-byte buffer

Out:	if successful:

CF clear

the buffer pointed to by ES:EDI contains descriptor

if failed

CF set

AX = error code

8022h - invalid selector

2.10 - DPMI function 000Ch - Set Descriptor

=============================================

Copies the contents of an 8-byte buffer into the descriptor for the specified

selector.

In:	AX = 000Ch

BX = selector

ES:EDI= selector:offset of 8-byte buffer containing descriptor

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8021h - invalid value (access rights/type byte invalid)

8022h - invalid selector

8025h - invalid linear address (descriptor references

a linear address range outside that allowed

for DPMI clients)

Notes:	a) The descriptor's access rights/type byte (byte 5) follows

the same format and restrictions as the access rights/type

parameter (in CL) for the Set Descriptor Access Rights

DPMI function 0009h, and the descriptor's extended access

rights/type byte (byte 6) follows the same format and

restrictions as the extended access rights/type parameter

(in CH) for the same function, except the low-order 4 bits

(marked "reserved") are used to set the upper 4 bits of the

descriptor's limit.

b) DOS/32 Advanced DPMI and DPMI 1.0 hosts will reload any

segment registers which contain the selector specified in

register BX.

2.11 - DPMI function 000Eh - Get Multiple Descriptors

=======================================================

Copies one or more descriptor table entries into a client buffer.

In:	AX = 000Eh

CX = number of descriptors to copy

ES:EDI= selector:offset of a buffer in the following format:

Offset   Length   Contents

00h      2        Selector #1 (set by client)

02h      8        Descriptor #1 (returned by host)

0Ah      2        Selector #2 (set by client)

0Ch      8        Descriptor #2 (returned by host)

.        .        .

.        .        .

.        .        .

Out:	if successful:

CF clear

the buffer contains copies of the descriptors for the specified

selectors

if failed:

CF set

AX = error code

CX = number of descriptors successfully copied

8022h - invalid selector

Notes:	a) If an error occurs because of an invalid selector or

descriptor, the function returns the number of

descriptors which were successfully copied in CX.  All of

the descriptors which were copied prior to the one that

failed are valid.

2.12 - DPMI function 000Fh - Set Multiple Descriptors

=======================================================

Copies one or more descriptors from a client buffer into the descriptor table.

In:	AX = 000Fh

CX = number of descriptors to copy

ES:EDI= selector:offset of a buffer in the following format:

Offset   Length   Contents

00h      2        Selector #1

02h      8        Descriptor #1

0Ah      2        Selector #2

0Ch      8        Descriptor #2

.        .        .

.        .        .

.        .        .

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

CX = number of descriptors successfully copied

8021h - invalid value (access rights/type bytes invalid)

8022h - invalid selector

8025h - invalid linear address (descriptor references a linear

address range outside that allowed for DPMI clients)

Notes:	a) If an error occurs because of an invalid selector or

descriptor, the function returns the number of

descriptors which were successfully copied in CX. All of

the descriptors which were copied prior to the one that

failed are valid. All descriptors from the invalid entry

to the end of the table are not updated.

b) The descriptor's access rights/type byte (byte 5) follows

the same format and restrictions as the access rights/type

parameter (in CL) for the Set Descriptor Access Rights

DPMI function 0009h, and the descriptor's extended access

rights/type byte (byte 6) follows the same format and

restrictions as the extended access rights/type parameter

(in CH) for the same function, except the low-order 4 bits

(marked "reserved") are used to set the upper 4 bits of the

descriptor's limit.

c) DOS/32 Advanced DPMI and DPMI 1.0 hosts will reload any

segment registers which contain the selector specified in

register BX.

2.13 - DPMI function 0100h - Allocate DOS Memory Block

========================================================

Allocates a block of memory from the DOS memory pool, ie memory below the

1 MB boundary that is controlled by DOS. Such memory blocks are typically

used to exchange data with real mode programs, TSRs, or device drivers.

The function returns both the real mode segment base address of the block

and one or more descriptors that can be used by protected mode applications

to access the block.

In:	AX = 0100h

BX = number of (16-byte) paragraphs desired

Out:	if successful:

CF clear

AX = real mode segment base address of allocated block

DX = selector for allocated block

if failed

CF set

AX = error code

BX = size of largest available block in paragraphs

0007h - memory control blocks damaged

0008h - insufficient memory

8011h - descriptor unavailable

Notes:	a) The selector returned by this function will be a writeable

data selector with the base set to the linear address of the

allocated memory block and limit set to the size of the memory

block.

b) Client programs should never modify or free any

descriptors allocated by this function. The Free DOS

Memory Block DPMI function 0101h will deallocate the

descriptors automatically.

c) The DOS allocation function (INT 21h function 48h) is

used.

2.14 - DPMI function 0101h - Deallocate DOS Memory Block

==========================================================

Frees a memory block that was previously allocated with the Allocate DOS

Memory Block DPMI function 0100h.

In:	AX = 0101h

DX = selector of block to be freed

Out:	if successful:

CF clear

if function unsuccessful

CF set

AX = error code

0007h - memory control blocks damaged

0009h - incorrect memory segment specified

8022h - invalid selector

Notes:	a) All descriptors allocated for the memory block are

automatically freed by this function, and are no longer

valid after this function returns.

b) Under DOS/32 Advanced DPMI and DPMI 1.0 hosts, any segment

registers which contain a selector being freed are zeroed

by this function.

2.15 - DPMI function 0102h - Resize DOS Memory Block

======================================================

Changes the size of a memory block that was previously allocated with the

Allocate DOS Memory Block DPMI function 0100h.

In:	AX = 0102h

BX = new block size in (16-byte) paragraphs

DX = selector of block to modify

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

BX = maximum possible block size (paragraphs)

0007h - memory control blocks damaged

0008h - insufficient memory

0009h - incorrect memory segment specified

8011h - descriptor unavailable

8022h - invalid selector

Notes:	a) Requests to increase the size of an existing DOS memory

block may fail due to subsequent DOS memory block allocations

causing fragmentation of DOS memory, or insufficient remaining

DOS memory.

b) Under DOS/32 Advanced and DPMI 1.0 hosts, any segment

registers which contain a selector being modified are reloaded

by this function.

c) Client programs should never modify or free any

descriptors allocated by this function. The Free DOS

Memory Block DPMI function 0101h will deallocate the

descriptors automatically.

2.16 - DPMI function 0200h - Get Real Mode Interrupt Vector

=============================================================

Returns the contents of the current virtual machine's real mode interrupt

vector for the specified interrupt.

In:	AX = 0200h

BL = interrupt number

Out:	CF clear

CX:DX = segment:offset of real mode interrupt handler

Notes:	a) The value returned in CX is a real mode segment address,

not a selector. Attempts to place this value into a segment

register in protected mode may cause a General Protection

(GP) fault exception.

2.17 - DPMI function 0201h - Set Real Mode Interrupt Vector

=============================================================

Sets the current virtual machine's real mode interrupt vector for the

specified interrupt.

In:	AX = 0201h

BL = interrupt number

CX:DX  = segment:offset of real mode interrupt handler

Out:	CF clear

Notes:	a) The address passed in CX must be a real mode segment

address, not a selector.  Consequently, the interrupt

handler must either reside in DOS memory (ie below the

1 MB boundary) or the client must allocate a real mode

callback address.  See DPMI functions 0100h and 0303h.

b) If the interrupt being hooked is a hardware interrupt,

the memory that the interrupt handler uses should be

locked.

c) DOS/32 Advanced DPMI will temporarely disable the

Null-Pointer Protection (if it was enabled) in order to

modify the first four interrupt vectors, INTs 00-03h.

2.18 - DPMI function 0202h - Get Processor Exception Handler Vector

=====================================================================

Returns the address of the current client's protected mode exception handler

for the specified exception number.

In:	AX = 0202h

BL = exception number (00-1Fh)

Out:	if successful:

CF clear

CX:EDX= selector:offset of exception handler

if failed:

CF set

AX = error code

8021h - invalid value (BL not in range 00-1Fh)

Notes:	a) The value returned in CX is a valid protected mode

selector, not a real mode segment address.

b) DOS/32 Advanced DPMI supports only first 16 exceptions.

Calls to this function with value in BL greater than 0Fh

will return CX:EDX = 0 (0000:00000000h).

c) DOS/32 Advanced DPMI does not handle exception 0Ch

(Stack Fault).

e) DOS/32 Advanced DPMI will read the exceptions 08-0Fh from

an internal buffer, not from IDT, when master PIC is mapped

on this interrupts (INTs 08-0Fh).

2.19 - DPMI function 0203h - Set Processor Exception Handler Vector

=====================================================================

Sets the address of a handler for a CPU exception or fault, allowing a

protected mode application to intercept processor exceptions (such as segment

not present faults) that are not handled by the DPMI host and would otherwise

generate a fatal error.

In:	AX = 0203h

BL = exception/fault number (00-1Fh)

CX:EDX= selector:offset of exception handler

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8021h - invalid value (BL not in range 00-1Fh)

8022h - invalid selector

Notes:	a) The value passed in CX should  be a valid protected mode

code (executable) selector, not a real mode segment address.

b) Every exception is first examined by the DPMI host.

If the host does not handle the exception, it reflects the

exception to the first handler in the protected mode

exception handler chain.

c) DOS/32 Advanced DPMI supports only first 16 exceptions.

Calls to this function with value in BL greater than 0Fh

will return CX:EDX = 0 (0000:00000000h).

d) DOS/32 Advanced DPMI does not handle exception 0Ch

(Stack Fault).

e) DOS/32 Advanced DPMI will install the exceptions 08-0Fh

in an internal buffer, not in IDT, when master PIC is mapped

on this interrupts (INTs 08-0Fh).

2.20 - DPMI function 0204h - Get Protected Mode Interrupt Vector

==================================================================

Returns the address of the current protected mode interrupt handler for the

specified interrupt.

In:	AX = 0204h

BL = interrupt number

Out:	CF clear

CX:EDX= selector:offset of exception handler

Notes:	a) The value returned in CX is a valid protected mode

selector, not a real mode segment address.

b) Client should use DPMI function 0202h to obtain

addresses of exception handlers, since interrupt and

exception handler's addresses may be different for

interrupts in range INT 00-0Fh.

2.21 - DPMI function 0205h - Set Protected Mode Interrupt Vector

==================================================================

Sets the address of protected mode handler for the specified interrupt into

the interrupt vector.

In:	AX = 0205h

BL = interrupt number

CX:EDX= selector:offset of exception handler

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8022h - invalid selector

Notes:	a) The value passed in CX  should be a valid protected mode

code selector, not a real mode segment address.

2.22 - DPMI function 0300h - Simulate Real Mode Interrupt

===========================================================

Simulates an interrupt in real mode.  The function transfers control to the

address specified by the real mode interrupt vector. The real mode handler

must return by executing an IRET.

In:	AX = 0300h

BL = interrupt number

BH = flags (must be 0)

CX = number of words to copy from protected mode to real mode stack

ES:EDI= selector:offset of real mode register data structure in

the following format:

Offset   Length  Contents

00h       4       EDI

04h       4       ESI

08h       4       EBP

0Ch       4       reserved, should be zero

10h       4       EBX

14h       4       EDX

18h       4       ECX

1Ch       4       EAX

20h       2       CPU status flags

22h       2       ES

24h       2       DS

26h       2       FS

28h       2       GS

2Ah       2       IP (reserved, ignored)

2Ch       2       CS (reserved, ignored)

2Eh       2       SP

30h       2       SS

Out:	if successful:

CF clear

ES:EDI= selector:offset of modified real mode register data structure

if failed:

CF set

AX = error code

8012h - linear memory unavailable (stack)

8013h - physical memory unavailable (stack)

8014h - backing store unavailable (stack)

8021h - invalid value (CX too large)

Notes:	a) The CS:IP in the real mode register data structure is

ignored by this function.  The appropriate interrupt

handler will be called based on the value passed in BL.

b) If the SS:SP fields in the real mode register data

structure are zero, a real mode stack will be provided by

the DPMI host.  Otherwise, the real mode SS:SP will be

set to the specified values before the interrupt handler

is called.

c) The flags specified in the real mode register data

structure will be pushed on the real mode stack's IRET

frame.  The interrupt handler will be called with the

interrupt and trace flags clear.

d) Values placed in the segment register positions of the

data structure must be valid for real mode; ie the

values must be paragraph addresses and not selectors.

e) All general register fields in the data structure are

DWORDs so that 32-bit registers can be passed to real

mode.

f) The target real mode handler must return with the stack

in the same state as when it was called.  This means that

the real mode code may switch stacks while it is running,

but must return on the same stack that it was called on

and  must return with an IRET.

g) When this function returns, the real mode register data

structure will contain the values that were returned by

the real mode interrupt handler. The CS:IP and SS:SP values

will be unmodified in the register data structure.

h) It is the caller's responsibility to remove any

parameters that were pushed on the protected mode stack.

2.23 - DPMI function 0301h - Call Real Mode Procedure With Far Return Frame

=============================================================================

Simulates a FAR CALL to a real mode procedure.  The called procedure must

return by executing a RETF (far return) instruction.

In:	AX = 0301h

BH = flags (must be 0)

CX = number of words to copy from protected mode to real mode stack

ES:EDI= selector:offset of real mode register data structure in

the following format:

Offset   Length  Contents

00h       4       EDI

04h       4       ESI

08h       4       EBP

0Ch       4       reserved, should be zero

10h       4       EBX

14h       4       EDX

18h       4       ECX

1Ch       4       EAX

20h       2       CPU status flags

22h       2       ES

24h       2       DS

26h       2       FS

28h       2       GS

2Ah       2       IP (reserved, ignored)

2Ch       2       CS (reserved, ignored)

2Eh       2       SP

30h       2       SS

Out:	if successful:

CF clear

ES:EDI= selector:offset of modified real mode register data structure

if failed:

CF set

AX = error code

8012h - linear memory unavailable (stack)

8013h - physical memory unavailable (stack)

8014h - backing store unavailable (stack)

8021h - invalid value (CX too large)

Notes:	a) The CS:IP in the real mode register data structure

specifies the address of the real mode procedure to call.

b) If the SS:SP fields in the real mode register data

structure are zero, a real mode stack will be provided by

the DPMI host. Otherwise, the real mode SS:SP will be

set to the specified values before the interrupt handler

is called.

c) Values placed in the segment register positions of the

data structure must be valid for real mode; ie the

values must be paragraph addresses and not selectors.

d) All general register fields in the data structure are

DWORDs so that 32-bit registers can be passed to real mode.

e) The target real mode procedure must return with the stack

in the same state as when it was called.  This means that

the real mode code may switch stacks while it is running,

but must return on the same stack that it was called on

and must exit with a RETF (far return) and should not

clear the stack of any other parameters that were passed

to it on the stack.

f) When this function returns, the real mode register data

structure will contain the values that were returned by

the real mode procedure. The CS:IP and SS:SP values

will be unmodified in the register data structure.

g) It is the caller's responsibility to remove any

parameters that were pushed on the protected mode stack.

2.24 - DPMI function 0302h - Call Real Mode Procedure With IRET Frame

=======================================================================

Simulates a FAR CALL with flags pushed on the stack to a real mode procedure.

The real mode routine must return by executing an IRET instruction.

In:	AX = 0302h

BH = flags (must be 0)

CX = number of words to copy from protected mode to real mode stack

ES:EDI= selector:offset of real mode register data structure in

the following format:

Offset   Length  Contents

00h       4       EDI

04h       4       ESI

08h       4       EBP

0Ch       4       reserved, should be zero

10h       4       EBX

14h       4       EDX

18h       4       ECX

1Ch       4       EAX

20h       2       CPU status flags

22h       2       ES

24h       2       DS

26h       2       FS

28h       2       GS

2Ah       2       IP (reserved, ignored)

2Ch       2       CS (reserved, ignored)

2Eh       2       SP

30h       2       SS

Out:	if successful:

CF clear

ES:EDI= selector:offset of modified real mode register data structure

if failed:

CF set

AX = error code

8012h - linear memory unavailable (stack)

8013h - physical memory unavailable (stack)

8014h - backing store unavailable (stack)

8021h - invalid value (CX too large)

Notes:	a) The CS:IP in the real mode register data structure

specifies the address of the real mode procedure to call.

b) If the SS:SP fields in the real mode register data

structure are zero, a real mode stack will be provided by

the DPMI host. Otherwise, the real mode SS:SP will be

set to the specified values before the interrupt handler

is called.

c) The flags specified in the real mode register data

structure will be pushed on the real mode stack's IRET

frame. The procedure will be called with the interrupt

and trace flags clear.

d) Values placed in the segment register positions of the

data structure must be valid for real mode; ie the

values must be paragraph addresses and not selectors.

e) All general register fields in the data structure are

DWORDs so that 32-bit registers can be passed to real mode.

f) The target real mode handler or procedure must return

with the stack in the same state as when it was called.

This means that the real mode code may switch stacks

while it is running, but must return on the same stack

that it was called on and must return with an IRET or

discard the flags from the stack with a RETF(2) .

g) When this function returns, the real mode register data

structure will contain the values that were returned by

the real mode procedure. The CS:IP and SS:SP values

will be unmodified in the register data structure.

h) It is the caller's responsibility to remove any

parameters that were pushed on the protected mode stack.

2.25 - DPMI function 0303h - Allocate Real Mode Callback Address

==================================================================

Returns a unique real mode segment:offset, known as a "real mode callback"

that will transfer control from real mode to a protected mode procedure.

Callback addresses obtained with this function can be passed by a protected

mode program to a real mode application, interrupt handler, device driver,

or TSR, so that the real mode program can call procedures within the

protected mode program or notify the protected mode program of an event.

In:	AX = 0303h

DS:ESI= selector:offset of protected mode procedure to call

ES:EDI= selector:offset of 32h-byte buffer for real mode register

data structure to be used when calling callback routine.

Out:	if successful:

CF clear

CX:DX = segment:offset of real mode callback

if failed:

CF set

AX = error code

8015h - callback unavailable

Notes:	a) A descriptor may be allocated for each callback to hold

the real mode SS descriptor. Real mode callbacks are a

limited system resource. A client should use the Free

Real Mode Callback Address DPMI function 0304h to release

a callback that it is no longer using.

b) The contents of the real mode register data structure is

not valid after the function call, but only at the time

of the actual callback.

2.26 - DPMI function 0304h - Free Real Mode Callback Address

==============================================================

Releases a real mode callback address that was previously allocated with

the Allocate Real Mode Callback Address DPMI function 0303h.

In:	AX = 0304h

CX:DX = real mode callback address to be freed

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8024h - invalid callback address

Notes:	a) Real mode callbacks are a limited system resource.

Client should release any callback that it is no longer

using.

2.27 - DPMI function 0305h - Get State Save/Restore Addresses

===============================================================

Returns the addresses of two procedures used to save and restore the state

of the current task's registers in the mode which is not currently executing.

In:	AX = 0305h

Out:	CF clear

AX = size of buffer in bytes required to save state

BX:CX = real mode address of routine used to save/restore state

SI:EDI = protected mode address of routine used to save/restore state

Notes:	a) The real mode address returned by this function in BX:CX

is called only in real mode to save/restore the state of

the protected mode registers.  The protected mode address

returned by this function in SI:EDI is called only in

protected mode to save/restore the state of the real mode

registers. Registers for the current mode can be saved by

simply pushing them on the stack.

b) Both of the state-save procedures are entered by a FAR

CALL with the following parameters:

AL = 0 to save state

= 1 to restore state

ES:EDI= (selector or segment):offset of state-save buffer

c) The state-save buffer must be at least as large as the

value returned in AX by DPMI function 0305h. The state

save/restore procedures do not modify any registers.

d) Some DPMI hosts will not require the state to be saved,

indicating this by returning a buffer size of zero in AX.

In such cases, the addresses returned by this function

can still be called, although they will simply return

without performing any useful function.

e) Clients do not need to call the state save/restore

procedures before using DPMI functions 0300h, 0301h,

or 0302h. The state save/restore procedures are provided

specifically for clients that use the raw mode switch

services.

f) A client can use the function to save its state in the

destination mode before switching modes using the raw

mode switch or issuing real-mode calls from a protected

mode hardware interrupt handler.

2.28 - DPMI function 0306h - Get Raw Mode Switch Addresses

============================================================

Returns addresses that can be called for low-level mode switching.

In:	AX = 0306h

Out:	CF clear

BX:CX = real-to-protected mode switch address

SI:EDI = protected-to-real mode switch address

Notes:	a) The address returned in BX:CX must only be called in real

mode to switch into protected mode.  The address returned

in SI:EDI must only be called in protected mode to switch

into real mode.

b) The mode switch procedures are entered by a FAR JMP to

the appropriate address with the following parameters:

AX	= new DS

CX	= new ES

DX	= new SS

(E)BX	= new (E)SP

SI	= new CS

(E)DI	= new (E)IP

The processor is placed into the desired mode, and the DS, ES, SS,

(E)SP, CS, and (E)IP registers are updated with the specified values;

in other words, execution of the client	continues in the requested

mode at the address provided in registers SI:EDI. The values

specified to be placed into the segment registers must be appropriate

for the destination mode; if invalid selectors are supplied when

switching into protected mode, an exception will occur.

The values in (E)AX, (E)BX, (E)CX, (E)DX, (E)SI, and (E)DI after the

mode switch are undefined. (E)BP will be preserved across the mode

switch call so it can be used as a pointer. The FS and GS segment

registers will contain zero after the mode switch.

If interrupts are disabled when the mode switch procedure is invoked,

they will not be re-enabled by the DPMI host (even temporarily).

c) It is up to the client to save and restore the state of

the task when using this function to switch modes. This

usually requires using the state save/restore procedures

whose addresses are returned by DPMI function 0305h.

d) Clients may find it more convenient to use DPMI functions

0300h, 0301h, and 0302h for mode switching than this function.

2.29 - DPMI function 0400h - Get Version

==========================================

Returns the version number of the DPMI Specification implemented by the

DPMI host. Clients can use this information to determine which function

calls are supported in the current environment.

In:	AX = 0400h

Out:	CF clear

AL = DPMI minor version (binary)

AH = DPMI major version (binary)

BX = flags

Bits	Significance

0	 0 = host is 16-bit DPMI implementation

1 = host is 32-bit (80386+) DPMI implementation

1	 0 = CPU returned to V86 mode for reflected interrupts

1 = CPU returned to real mode for reflected interrupts

2	 0 = virtual memory not supported

1 = virtual memory supported

3-15	 reserved

CL = processor type

03h = 80386

04h = 80486

05h = 80586 (Pentium)

06h = 80686 (Pentium Pro)

07h-FFh = reserved for future Intel processors

DL = current value of virtual slave PIC base interrupt

DH = current value of virtual master PIC base interrupt

Notes:	a) Under DPMI hosts, the major version number is returned in

DH and the minor version number is returned in DL. There

are two decimal digits for the minor version number with

the least-significant digit representing the revision

number of the minor version number. Under DPMI version

0.9 hosts, DH is returned as 0, and DL is returned as

decimal 90 (5Ah). In hypothetical DPMI version 2.3, DH

would be returned as 2 and DL would be returned as 30

(1Eh).

2.30 - DPMI function 0500h - Get Free Memory Information

==========================================================

Returns information about the amount of available physical memory, linear

address space, and disk space for page swapping. Since DPMI clients will

often run in multitasking environments, the information returned by this

function should only be considered as advisory.

In:	AX = 0500h

ES:EDI = selector:offset of 48-byte buffer

Out:	CF clear

the buffer is filled in with the following information:

Offset Length  Contents

00h	 4       Largest available free block in bytes

04h	 4       Maximum unlocked page allocation in pages

08h	 4       Maximum locked page allocation in pages

0Ch	 4       Linear address space size in pages

10h	 4       Total number of unlocked pages

14h	 4       Total number of free pages

18h	 4       Total number of physical pages

1Ch	 4       Free linear address space in pages

20h	 4       Size of paging file/partition in pages

24h	0Ch      Reserved, all bytes set to 0FFh

Notes:	a) Only the first field of the returned structure is

guaranteed to contain a valid value.  Any fields that are

not supported by the DPMI host will be set by the host to

-1 (0FFFFFFFFh) to indicate that the information is not

available.

b) The field at buffer offset 00h specifies the largest

block of contiguous linear memory in bytes that could be

allocated if the memory were to be allocated and left

unlocked.

c) The field at buffer offset 04h specifies the largest

number of pages that could be allocated (the value at

offset 00h divided by the page size).

d) The field at buffer offset 08h specifies the largest

block of memory in pages that could be allocated and then

locked.

e) The field at buffer offset 0Ch specifies the size of the

total linear address space in pages.  This value includes

all linear address space that has already been allocated.

f) The field at buffer offset 10h specifies the total number

of pages that are currently unlocked and could be paged out.

This value also contains any free pages.

g) The field at buffer offset 14h specifies the number of

physical pages that currently are not in use.

h) The field at offset 18h specifies the total number of

physical pages that the DPMI host manages.  This value

includes all free, locked, and unlocked physical pages.

i) The field at offset 20h specifies the size of the DPMI

host's paging partition or file in pages.

j) The size of the pages used by the DPMI host can be

obtained with the Get Page Size DPMI function 0604h.

2.31 - DPMI function 0501h - Allocate Memory Block

====================================================

Allocates and commits a block of linear memory.

In:	AX = 0501h

BX:CX = size of block (bytes, must be nonzero)

Out:	if successful:

CF clear

BX:CX = linear address of allocated memory block

SI:DI = memory block handle (used to resize and free block)

if failed:

CF set

AX = error code

8012h - linear memory unavailable

8013h - physical memory unavailable

8014h - backing store unavailable

8016h - handle unavailable

8021h - invalid value (BX:CX = 0)

Notes:	a) The allocated block is guaranteed to have at least

paragraph alignment. Under DOS/32 Advanced DPMI the blocks

will be DWORD aligned only.

b) This function does not allocate any descriptors for the

memory block.  It is the responsibility of the client to

allocate and initialize any descriptors needed to access

the memory with additional DPMI function calls.

c) Under DPMI hosts that support virtual memory, the memory

block will be allocated unlocked.  The client can lock

some or all of the memory after it is allocated with the

Lock Linear Region DPMI function 0600h.

2.32 - DPMI function 0502h - Free Memory Block

================================================

Frees a memory block that was previously allocated with the Allocate Memory

Block DPMI function 0501h.

In:	AX = 0502h

SI:DI = memory block handle

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8023h - invalid handle

Notes:	a) No descriptors are freed by this call.  It is the

client's responsibility to free any descriptors that it

previously allocated to map the memory block.

Descriptors should be freed before linear memory blocks.

2.33 - DPMI function 0503h - Resize Memory Block

==================================================

Changes the size of a memory block that was previously allocated with the

Allocate Memory Block function 0501h.

In:	AX = 0503h

BX:CX = new size of block (bytes, must be nonzero)

SI:DI = memory block handle

Out:	if successful:

CF clear

BX:CX = new linear address of memory block

SI:DI = new handle for memory block

if failed:

CF set

AX = error code

8012h - linear memory unavailable

8013h - physical memory unavailable

8014h - backing store unavailable

8016h - handle unavailable

8021h - invalid value (BX:CX = 0)

8023h - invalid handle (in SI:DI)

Notes:	a) After this function returns, the previous handle for the

memory block is invalid and should not be used.

b) The linear address and handle of the memory block may

change as a result of this call.

c) It is the client's responsibility to update any

descriptors that map the memory block with the new linear

address after resizing the block.

d) This function returns an error if the client attempts to

resize a memory block to zero bytes.

2.34 - DPMI function 050Ah - Get Memory Block Size and Base

=============================================================

Returns the size of a memory block that was previously allocated with

DPMI function 0501h.

In:	AX = 050Ah

SI:DI = memory block handle

Out:	if successful:

CF clear

SI:DI = size of memory block (bytes)

BX:CX = base address of memory block

if failed:

CF set

AX = error code

8023h - invalid handle

2.35 - DPMI function 0600h - Lock Linear Region

=================================================

Locks the specified linear address range.

In:	AX = 0600h

BX:CX = starting linear address of memory to lock

SI:DI = size of region to lock (bytes)

Out:	if successful:

CF clear

if function unsuccessful

CF set

AX = error code

8013h - physical memory unavailable

8017h - lock count exceeded

8025h - invalid linear address (unallocated pages)

Notes:	a) If the function returns an error, none of the memory has

been locked.

b) If the specified region overlaps part of a page at the

beginning or end of the region, the page(s) will be

locked.

c) This function may be called more than once for a given

page; the DPMI host maintains a lock count for each page.

d) This function is ignored by DPMI implementations that do

not support virtual memory; the function will return the

Carry flag clear to indicate success, but has no other

effect. DPMI hosts which support virtual memory may also

choose to ignore this function, but such hosts must be

able to handle page faults transparently at arbitrary

points during a client's execution, including within

interrupt and exception handlers.

e) Since DOS/32 Advanced does not support virtual memory

this function is implemented for compatibility reasons only.

2.36 - DPMI function 0601h - Unlock Linear Region

===================================================

Unlocks a linear address range that was previously locked using the

Lock Linear Region DPMI function 0600h.

In:	AX = 0601h

BX:CX = starting linear address of memory to unlock

SI:DI = size of region to unlock (bytes)

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8002h - invalid state (page not locked)

8025h - invalid linear address (unallocated pages)

Notes:	a) If the function returns an error, none of the memory has

been unlocked.

b) If the specified region overlaps part of a page at the

beginning or end of the region, the page(s) will be

unlocked.

c) A lock count is maintained for each locked page; the page

is not unlocked until the lock count is decremented to

zero.

d) This function is ignored by DPMI implementations that do

not support virtual memory; the function will return the

Carry flag clear to indicate success, but has no other

effect. DPMI hosts which support virtual memory may also

choose to ignore this function, but such hosts must be

able to handle page faults transparently at arbitrary

points during a client's execution, including within

interrupt and exception handlers.

e) Since DOS/32 Advanced does not support virtual memory

this function is implemented for compatibility reasons only.

2.37 - DPMI function 0604h - Get Page Size

============================================

Returns the size of a single memory page in bytes.

In:	AX = 0604h

Out:	CF clear

BX:CX = page size in bytes

2.38 - DPMI function 0702h - Mark Page as Demand Paging Candidate

===================================================================

Notifies the DPMI host that a range of pages may be placed at the head of

the page-out candidate list, forcing these pages to be replaced ahead of

other pages even if the memory has been accessed recently. The contents

of the pages will be preserved.

In:	AX = 0702h

BX:CX = starting linear address of pages to mark as paging candidates

SI:DI = size of region to mark (bytes)

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8025h - invalid linear address (range unallocated)

Notes:	a) DOS/32 Advanced DPMI will perform no action on this function

and will always return carry flag clear.

2.39 - DPMI function 0703h - Discard Page Contents

====================================================

Discards the entire contents of a given linear memory range. This function

is used when a memory object (such as a data structure) that occupies a given

area of memory is no longer needed, so that the area will not be paged to disk

unnecessarily.  The contents of the discarded region will be undefined.

In:	AX = 0703h

BX:CX = starting linear address of pages to discard

SI:DI = size of region to discard (bytes)

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8025h - invalid linear address (range unallocated)

Notes:	a) DOS/32 Advanced DPMI will perform no action on this function

and will always return carry flag clear.

2.40 - DPMI function 0800h - Physical Address Mapping

=======================================================

Converts a physical address into a linear address. This function allows

device drivers running under DPMI hosts which use paging to reach physical

memory that is associated with their devices above the 1 MB boundary.

Examples of such devices are the Weitek numeric coprocessor (usually mapped

at 3 GB), buffers that hold scanner bitmaps, and high-end displays that can

be configured to make display memory appear in extended memory.

In:	AX = 0800h

BX:CX = physical address of memory

SI:DI = size of region to map (bytes)

Out:	if successful:

CF clear

BX:CX = linear address that can be used to access the physical memory

if failed:

CF set

AX = error code

8003h - system integrity (DPMI host memory region)

8021h - invalid value (address is below 1 MB boundary)

Notes:	a) It is the caller's responsibility to allocate and

initialize a descriptor for access to the memory.

b) This function should only be used by clients that

absolutely require direct access to a memory mapped

device at physical addresses above 1 MB. Clients should

not use this function to access memory below the 1 MB

boundary (the real mode addressable region). See also

DPMI functions 0002h.

c) When this function is called, the DPMI host either

creates page table entries that directly map the physical

addresses requested and returns the linear address of the

created page table entries, or else just returns the

linear address of the memory region that is already used

to map the requested device. For example, if the client

attempts to map a Weitek coprocessor and the host already

has a linear region set up to map the Weitek chip and

virtualize it, it would simply return the linear address

of the existing region.  If the host does not virtualize

the Weitek chip, it would create 16 page table entries

that map the 64 KB Weitek address space and return a

linear address corresponding to the new page table

entries.

d) If the host is not virtualizing the device, it will

disable any memory caching on the mapped pages; in

particular, on the 80486+ the host will set the PCD (page

cache disable) bit in the page table entries.

e) Programs and device drivers which need to perform DMA I/O

to physical addresses in a virtualized hardware

environment should use the Virtual DMA Services.

2.41 - DPMI function 0801h - Free Physical Address Mapping

============================================================

Releases a mapping of physical to linear addresses that was previously

obtained with the Physical Address Mapping DPMI function 0800h.

In:	AX = 0801h

BX:CX = linear address returned by physical address mapping call

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8025h - invalid linear address

Notes:	a) The client should call this function when it is finished

using a device previously mapped to linear addresses with

the Physical Address Mapping DPMI function 0800h.

2.42 - DPMI function 0900h - Get and Disable Virtual Interrupt State

======================================================================

Disables the virtual interrupt flag and returns the previous state of the

virtual interrupt flag.

In:	AX = 0900h

Out:	CF clear

Virtual interrupts disabled

AL = 0 if virtual interrupts were previously disabled

= 1 if virtual interrupts were previously enabled

Notes:	a) AH is not changed by this function. Therefore, the

previous state can be restored by simply executing

another INT 31h. See DPMI function 0901h.

b) A client that does not need to know the prior interrupt

state can execute the CLI instruction rather than calling

this function.  The instruction may be trapped by the

host and should be assumed to  be very slow.

2.43 - DPMI function 0901h - Get and Enable Virtual Interrupt State

=====================================================================

Enables the virtual interrupt flag and returns the previous state of the

virtual interrupt flag.

In:	AX = 0901h

Out:	CF clear

Virtual interrupts enabled

AL = 0 if virtual interrupts were previously disabled

= 1 if virtual interrupts were previously enabled

Notes:	a) AH is not changed by this function. Therefore, the

previous state can be restored by simply executing

another INT 31h. See DPMI function 0900h.

b) A client that does not need to know the prior interrupt

state can execute the STI instruction rather than calling

this function. The instruction may be trapped by the

host and should be assumed to be very slow.

2.44 - DPMI function 0901h - Get Virtual Interrupt State

==========================================================

Returns the current state of the virtual interrupt flag.

In:	AX = 0902h

Out:	CF clear

AL = 0 if virtual interrupts are disabled

= 1 if virtual interrupts are enabled

Notes:	a) This function should be used in preference to the PUSHF

instruction to examine the interrupt flag, because the

PUSHF instruction returns the physical interrupt flag

rather than the virtualized (per-client) interrupt flag.

On some DPMI hosts, the physical interrupt flag will

always be enabled, even when hardware interrupts are not

being passed through to the client.

2.45 - DPMI function 0A00h - Get Vendor-Specific API Entry Point

==================================================================

Returns an address which can be called to use host-specific extensions to the

standard set of DPMI functions.

In:	AX = 0A00h

DS:ESI = selector:offset of ASCIIZ (null-terminated) string which

identifies the DPMI host vendor

Out:	if successful:

CF clear

ES:EDI = (code)selector:offset of extended API entry point

DS, FS, GS, EAX, EBX, ECX, EDX, ESI, and EBP may be modified.

if failed:

CF set

AX = error code

8001h - unsupported function (extension not found)

Notes:	a) The null-terminated string specifies the host-specific

vendor name or some other unique identifier to obtain a

specific extension entry point. The string comparison

used to look up the API entry point is case-sensitive.

b) Clients must use a FAR CALL to reach the extended API

entry point.

c) All extended API parameters are specified by the vendor.

d) DOS/32 Advanced recognizes two ID-strings:

1) "RATIONAL DOS/4G",\0

2) "SUNSYS DOS/32A",\0

When calling this function with DS:ESI pointing to the first

ID-string, ie "RATIONAL DOS/4G", DOS/32 Advanced will return

in ES:EDI a pointer to an entry point to the DOS/4G(W) DPMI

API extensions.

*) Please note that DOS/4G(W) DPMI API extensions are not supported

by this version of DOS/32 Advanced DPMI. A call to DOS/4G(W) extended

DPMI API entry point will result in a fatal error which will terminate

the application immediadely and return to DOS.

When calling this function with DS:ESI pointing to the second

ID-string, ie "SUNSYS DOS/32A", DOS/32 Advanced will return

in ES:EDI a pointer to an entry point to the DOS/32 Advanced

DPMI API extensions, and an additional information in the

following registers:

AX = DOS Extender version number

AL = minor version number (binary)

AH = major version number (binary)

BL = DPMI Kernel configuration bits

BH = System software type:

00h = Clean

01h = XMS

02h = VCPI

03h = DPMI

CL = CPU type:

03h = 80386

04h = 80486

05h = 80586 (Pentium)

06h = 80686 (Pentium Pro)

07h-0FFh = reserved

CH = FPU type:

00h = none (no co-processor is present)

01h = 8087

02h = 80287

03h = 80387

04h = 80486 with numeric coprocessor

05h = 80586 (Pentium)

06h = 80686 (Pentium Pro)

DX = PIC values (unrelocated)

ES:EDI = (code)selector:offset of DOS/32 Advanced DPMI API extensions

DS:ESI, EBP, FS, GS = unmodified

Please refer to the section 2.49 - Vendor Specific DPMI API Extensions

to get full information about DOS/32 Advanced extended DPMI API

functions.

Additional information about value returned in BL register and

about DOS/32 Advanced DPMI Kernel configuration can be found in

the document "DOS/32 Advanced Technical Reference".

2.46 - DPMI function 0E00h - Get Coprocessor Status

=====================================================

Returns information about whether or not a numeric coprocessor exists, the

type of coprocessor available (if any), and whether or not the host or client

is providing coprocessor emulation.

In:	AX = 0E00h

Out:	if successful:

CF clear

AX = coprocessor status

Bit    Significance

0	MPv (MP bit in the virtual MSW/CR0)

0 = numeric coprocessor is disabled for this client

1 = numeric coprocessor is enabled for this client

1	EMv (EM bit in the virtual MSW/CR0)

0 = client is not emulating coprocessor instructions

1 = client is emulating coprocessor instructions

2	MPr (MP bit from the actual MSW/CR0)

0 = numeric coprocessor is not present

1 = numeric coprocessor is present

3	EMr (EM bit from the actual MSW/CR0)

0 = host is not emulating coprocessor instructions

1 = host is emulating coprocessor instructions

4-7    coprocessor type:

00h = no coprocessor

01h = 8087

02h = 80287

03h = 80387

04h = 80486 with numeric coprocessor

05h = 80586 (Pentium)

06h = 80686 (Pentium Pro)

07h-0Fh  reserved for future numeric processors

8-15   not applicable

if failed:

CF set

Notes:	a) If the real EM (EMr) bit is set, the host is supplying or

is capable of supplying floating point emulation.

b) If the MPv bit is not set, the host may not need to save

the coprocessor state for this virtual machine to improve

system performance.

c) MPr bit setting should be consistent with the setting of

coprocessor type information. Ignore MPr bit information

if it is in conflict with the coprocessor type information.

d) If the virtual EM (EMv) bit is set, the host delivers all

coprocessor exceptions to the client, and the client is

performing its own floating point emulation (whether or

not a coprocessor is present or the host also has a floating

point emulator). In other words, if the EMv bit is set, the

host sets the EM bit in the real CR0 while the virtual machine

is active, and reflects coprocessor not present faults (INT 07h)

to the virtual machine.

e) A client can determine the CPU type with DPMI function 0400h

but a client should not draw any conclusions about the presence

or absence of a coprocessor based on the CPU type alone.

f) DOS/32 Advanced DPMI will not distinguish between 80387 and

80487 (80486 with build-in FPU). That is, if the CPU installed

in machine is 80486SX, and an 80387 FPU is present, DOS/32

Advanced DPMI will report the presence of an 80487.

g) DOS/32 Advanced DPMI does not have a build-in FPU emulator.

That is, when client enables FPU emulation (using Set Coprocessor

Emulation DPMI function 0E01h), but does not handle the exception

07h (Coprocessor not available), the first instruction executed

by the CPU that uses FPU will cause exception 07h which will be

trapped by DOS/32 Advanced which in its turn will terminate the

client and return to DOS with an error message.

2.47 - DPMI function 0E01h - Set Coprocessor Emulation

========================================================

Enables or disables the numeric coprocessor for this virtual machine and the

reflection of coprocessor exceptions to the client.

In:	AX = 0E01h

BX = coprocessor bits

Bit    Significance

0	new value of MPv bit for client's virtual CR0

0 = disable numeric coprocessor for this client

1 = enable numeric coprocessor for this client

1	new value of EMv bit for client's virtual CR0

0 = client will not supply coprocessor emulation

1 = client will supply coprocessor emulation

2-15	not applicable

Out:	if successful:

CF clear

if failed:

CF set

AX = error code

8026h - invalid request (client requested disabling

coprocessor on a processor which does not support it)

Notes:	a) If the MPv bit is not set, the host may not need to save

the coprocessor state for this virtual machine to improve

system performance.

b) If the virtual EM (EMv) bit is set, the host delivers all

coprocessor exceptions to the client, so that the client

can provide its own floating point emulation (whether or

not a coprocessor is present or the host also has a

floating point emulator). In other words, if the EMv bit

is set, the host sets the EM bit in the real CR0 while

the client is active, and reflects coprocessor not

present faults (Int 7) to the client.

c) Floating point emulation can be tested on a system with a

numeric coprocessor by using this function to enable

client handling of coprocessor exceptions and disable the

coprocessor.

d) The client should use DPMI function 0203h to register

an exception handler for Coprocessor not Present faults

(INT 07h) prior to setting the EMv bit with this function.

e) A client can determine the CPU type with DPMI function 0400h

and the presence or absence of a coprocessor with DPMI function

0E00h. The client should not draw any conclusions about the presence

or absence of a coprocessor based on the CPU type alone.

f) DOS/32 Advanced DPMI does not have a build-in FPU emulator.

That is, when client enables FPU emulation (using Set Coprocessor

Emulation DPMI function 0E01h), but does not handle the exception

07h (Coprocessor not available), the first instruction executed

by the CPU that uses FPU will cause exception 07h which will be

trapped by DOS/32 Advanced which in its turn will terminate the

client and return to DOS with an error message.

2.48 - DPMI function 0EEFFh - Get DOS Extender Info

=====================================================

Returns information about the DOS extender.

In:	AX = 0EEFFh

Out:	if successful:

CF clear

EAX = "PMDW" (504D4457h)

CL = CPU type:

03h = 80386

04h = 80486

05h = 80586 (Pentium)

06h = 80686 (Pentium Pro)

07h-0FFh = reserved

CH = System software type:

00h = Clean

01h = XMS

02h = VCPI

03h = DPMI

DL = DOS Extender minor version (binary)

DH = DOS Extender major version (binary)

ES:EBX = selector:offset of ASCIIZ copyright string

if failed:

CF set

AX = error code

8001h - unsupported function

Notes:	a) This function is not a part of DPMI 0.9 specification and

is invented for compatibility with PMODE/W DOS Extender only.

b) Under DOS/32 Advanced the pointer ES:EBX will point to

an empty string (to a byte 00h).

2.49 - DOS/32 Advanced vendor-specific DPMI functions

*******************************************************

The entry point to the vendor-specific DPMI functions can be obtained

through the DPMI call 0A00h with DS register containing the selector of the

ID-string and ESI register containing the offset. The entry point code

selector will be returned in ES and the offset of the entry point routine

will be placed in EDI. To access the vendor-specific DPMI API functions

you must issue a far call to the address pointed to by ES:EDI.

The ASCIIZ ID-string for DOS/4G extensions is: "RATIONAL DOS/4G",\0

(Please note that DOS/4G API extensions have not been implemented in this

version of DOS/32 Advanced DOS Extender. Any call to DOS/4G API, ie to the

address returned in ES:EDI will terminate the program immediately and display

an error message on the screen.)

The ASCIIZ ID-string for DOS/32 Advanced extensions is: "SUNSYS DOS/32A",\0

To call DOS/32 Advanced DPMI API functions you must place the function

number in AL register. On return the carry flag will be cleared, if the

function was successful. Otherwise the carry flag will be set and no registers

will be modified. Please note that none of API functions will perform any

kind of action on the system, instead they will simply return pointers and

variables from the DPMI kernel code that can be altered by the user himself.

WARNING: The following functions will give the user an access to the internal

variables inside DOS/32 Advanced DPMI kernel. Setting this variables to

improper values may cause the DOS Extender to malfunction and/or result in

loss of data. In other words: you may take a look at it, but touch it only

if you're sure it won't bite.

2.50 - API function 00h - Get Access to GDT and IDT

=====================================================

In:	AL = 00h

Out:	EBX = writeable data selector

ECX = GDT limit

EDX = IDT limit

ESI = pointer to GDT

EDI = pointer to IDT

Notes:	a) The selector returned in (E)BX is a writeable data selector

that should be used to access needed variables/addresses.

2.51 - API function 01h - Get Access to Page Tables

=====================================================

In:	AL = 01h

Out:	EBX = writeable data selector

ECX = number of allocated page tables

EDX = number of allocated page tables for physical address mapping

ESI = pointer to 0-th page table

EDI = pointer to first page table for physical address mapping

Notes:	a) The selector returned in (E)BX is a writeable data selector

that should be used to access needed variables/addresses.

b) This function will return the correct values under VCPI only.

2.52 - API function 02h - Get Access to Internal Interrupt Buffers

====================================================================

In:	AL = 02h

Out:	EBX = writeable data selector

ECX = number of interrupt buffers (3)

EDX = number of interrupts in buffer (16)

ESI = pointer to interrupt switches

EDI = pointer to 1-st interrupt buffer

Notes:	a) The selector returned in (E)BX is a writeable data selector

that should be used to access needed variables/addresses.

2.53 - API function 03h - Get Access to Extended Memory Blocks

================================================================

In:	AL = 03h

Out:	EBX = writeable data selector

ECX = total size of allocated extended memory

EDX = pointer to allocated extended memory (first byte)

ESI = pointer to top of allocated extended memory (last byte)

Notes:	a) The selector returned in (E)BX is a writeable data selector

that should be used to access needed variables/addresses.

2.54 - API function 04h - Get Access to Real Mode Virtual Stacks

==================================================================

In:	AL = 04h

Out:	EBX = writeable data selector

ECX = size of one real mode virtual stack

EDX = pointer to the current top of real mode virtual stack

ESI = pointer to base of real mode virtual stacks

Notes:	a) The selector returned in (E)BX is a writeable data selector

that should be used to access needed variables/addresses.

2.55 - API function 05h - Get Access to Protected Mode Virtual Stacks

=======================================================================

In:	AL = 05h

Out:	EBX = writeable data selector

ECX = size of one protected mode virtual stack

EDX = pointer to the current top of protected mode virtual stack

ESI = pointer to base of protected mode virtual stacks

Notes:	a) The selector returned in (E)BX is a writeable data selector

that should be used to access needed variables/addresses.

Please refer to the document "DOS/32 Advanced Technical Reference" to

learn more about DOS/32 Advanced DPMI API extensions and kernel variables.

2.60 - DPMI error codes

*************************

If a DPMI function was unsuccessful, carry flag (CF) will be set and

an error code will be placed in register AX. The following is a list of

error codes returned by a DPMI host.

8001h - Unsupported function

Returned in response to any function call which is not implemented by

this host, because the requested function is either undefined or

optional.

8002h - Invalid state

Some object is in the wrong state for the requested operation.

8003h - System integrity

The requested operation would endanger system integrity, eg, a request

to map linear addresses onto system code or data.

8004h - Deadlock

Host detected a deadlock situation.

8005h - Request cancelled

A pending serialization request was cancelled.

8010h - Resource Unavailable

The DPMI host cannot allocate internal resources to complete an

operation.

8011h - Descriptor unavailable

Host is unable to allocate a descriptor.

8012h - Linear memory unavailable

Host is unable to allocate the required linear memory.

8013h - Physical memory unavailable

Host is unable to allocate the required physical memory.

8014h - Backing store unavailable

Host is unable to allocate the required backing store.

8015h - Callback unavailable

Host is unable to allocate the required callback address.

8016h - Handle unavailable

Host is unable to allocate the required handle.

8017h - Lock count exceeded

A locking operation exceeds the maximum count maintained by the host.

8018h - Resource owned exclusively

A request for serialization of a shared memory block could not be

satisfied because it is already serialized exclusively by another

client.

8019h - Resource owned shared

A request for exclusive serialization of a shared memory block could

not be satisfied because it is already serialized shared by another

client.

8021h - Invalid value

A numeric or flag parameter has an invalid value.

8022h - Invalid selector

A selector does not correspond to a valid descriptor.

8023h - Invalid handle

A handle parameter is invalid.

8024h - Invalid callback

A callback parameter is invalid.

8025h - Invalid linear address

A linear address range (either supplied as a parameter or implied by

the call) is invalid.

8026h - Invalid request

The request is not supported by the underlying hardware.

3.0 - Extended DOS functions - Overview

*****************************************

The following is a list of all DOS functions that have been extended and

made available to your program when calling them from protected mode. Most of

them require a pointer that is to be converted to real mode values and passed

down to real mode DOS. The extended DOS functions are available through

software interrupt 21h (INT 21h). Function number is placed in AH register.

If the function was successful, carry flag will be clear on return, otherwise

carry flag will be set, and an error code will be placed in AX register.

Please note that most of real mode DOS functions that do not require pointer

translation will work in protected mode as well.

3.01 - DOS function 09h - Write String

========================================

In:	AH = 09h

DS:EDX = pointer to '$' terminated string

Out:	-

3.02 - DOS function 0Ah - Buffered Input

==========================================

In:	AH = 0Ah

DS:EDX = pointer to buffer

Out:	buffer filled with user input data

Notes:	a) CTRL-C and CTRL-BREAK are checked and INT 23h is called if

either is detected.

3.03 - DOS function 1Ah - Set DTA Address

===========================================

In:	AH = 1Ah

DS:EDX = pointer to DTA buffer DTA

Out:	-

Notes:	a) DOS/32 Advanced allocates an internal DTA buffer in conventional

memory. Each extended DOS function that accesses DTA will copy data

from your DTA (pointed by DS:EDX) to this buffer, call real mode DOS

function, and copy the updated data from the buffer back to your DTA.

3.04 - DOS function 1Bh - Get Default Drive Info

==================================================

In:	AH = 1Bh

Out:	AL = sectors per cluster

ECX = bytes per sector

EDX = total number of clusters on drive

DS:EBX = pointer to media ID byte

Notes:	a) The selector returned in DS register will be zero selector, ie

readable/writeable data selector with Base=0. This is an internal

to DOS/32 Advanced selector which may NOT be freed or modified

under any circumstances.

3.05 - DOS function 1Ch - Get Specific Drive Info

===================================================

In:	AH = 1Ch

DL = drive number

Out:	if successful:

AL = sectors per cluster

ECX = bytes per sector

EDX = total number of clusters on drive

DS:EBX = pointer to media ID byte

if failed:

AL = 0FFh (invalid drive in DL)

Notes:	a) The selector returned in DS register will be zero selector, ie

readable/writeable data selector with Base=0. This is an internal

to DOS/32 Advanced selector which may NOT be freed or modified

under any circumstances.

3.06 - DOS function 1Fh - Get Default Drive Parameter Block Address

=====================================================================

In:	AH = 1Fh

Out:	if successful:

AL = 00h

DS:EBX = pointer to DPB buffer

if failed:

AL = 0FFh (invalid drive)

<Notes:	a) The selector returned in DS register will be zero selector, ie

readable/writeable data selector with Base=0. This is an internal

to DOS/32 Advanced selector which may NOT be freed or modified

under any circumstances.

3.07 - DOS function 25h - Set Interrupt Vector

================================================

In:	AH = 25h

AL = INT vector

DS:EDX = selector:offset of interrupt handler

Out:	if successful:

CF clear

if failed:

CF set

Notes:	a) This function sets a protected mode interrupt vector

and is equivalent to DPMI function 0205h.

b) This function will indicate an error (set carry flag)

if the selector passed in DS was an invalid selector.

3.08 - DOS function 2Fh - Get DTA buffer

==========================================

In:	AH = 2Fh

Out:	ES:EBX = pointer to DTA buffer

Notes:	a) This function returns the pointer that was previously set by

DOS function 1Ah, or the pointer to the default buffer that was

allocated by DOS/32 Advanced if DOS function 1Ah was never called.

3.09 - DOS function 32h - Get Specific Drive Parameter Block

==============================================================

In:	AH = 32h

DL = drive number

Out:	if successful:

AL = 00h

DS:EBX = pointer to DPB

if failed:

AL = 0FFh (invalid drive in DL)

Notes:	a) The selector returned in DS register will be zero selector, ie

readable/writeable data selector with Base=0. This is an internal

to DOS/32 Advanced selector which may NOT be freed or modified

under any circumstances.

3.10 - DOS function 34h - Get InDOS Flag Address

==================================================

In:	AH = 34h

Out:	ES:EBX = address of InDOS flag

Notes:	a) The selector returned in ES register will be zero selector, ie

readable/writeable data selector with Base=0. This is an internal

to DOS/32 Advanced selector which may NOT be freed or modified

under any circumstances.

3.11 - DOS function 35h - Get Interrupt Vector

================================================

In:	AH = 35h

AL = INT vector

Out:	ES:EBX = selector:offset of interrupt handler

Notes:	a) This function returns a protected mode interrupt vector

and is equivalent to DPMI function 0204h.

3.12 - DOS function 39h - Create Directory

============================================

In:	AH = 39h

DS:EDX = pointer to ASCIIZ directory name

Out:	if successful:

CF clear

if failed:

CF set

EAX = DOS error code

3.13 - DOS function 3Ah - Remove Directory

============================================

In:	AH = 3Ah

DS:EDX = pointer to ASCIIZ directory name

Out:	if successful:

CF clear

if failed:

CF set

EAX = DOS error code

3.14 - DOS function 3Bh - Change Directory

============================================

In:	AH = 3Bh

DS:EDX = pointer to ASCIIZ directory name

Out:	if successful:

CF clear

if failed:

CF set

EAX = DOS error code

3.15 - DOS function 3Ch - Create File

=======================================

In:	AH = 3Ch

ECX = file attributes

DS:EDX = pointer to ASCIIZ file name

Out:	if successful:

CF clear

EAX = file handle

if failed:

CF set

EAX = DOS error code

3.16 - DOS function 3Dh - Open File

=====================================

In:	AH = 3Dh

AL = access mode

DS:EDX = pointer to ASCIIZ file name

Out:	if successful:

CF clear

EAX = file handle

if failed:

CF set

EAX = DOS error code

3.17 - DOS function 3Fh - Read from File

==========================================

In:	AH = 3Fh

EBX = file handle

ECX = number of bytes to read (size)

DS:EDX = pointer to buffer to read to (addr)

Out:	if successful:

CF clear

EAX = number of bytes read

if failed:

CF set

EAX = DOS error code

3.18 - DOS function 40h - Write to File

=========================================

In:	AH = 40h

EBX = file handle

ECX = number of bytes to write (size)

DS:EDX = pointer to buffer to write from (addr)

Out:	if successful:

CF clear

EAX = number of bytes written

if failed:

CF set

EAX = DOS error code

3.19 - DOS function 41h - Delete File

=======================================

In:	AH = 41h

DS:EDX = pointer to ASCIIZ file name

Out:	if succsessful:

CF clear

if failed:

CF set

EAX = DOS error code

3.20 - DOS function 42h - Set Current File Position

=====================================================

In:	AH = 42h

AL = origin of move (00h = start, 01h = current, 02h = end)

ECX:EDX = file position

Out:	if successful:

CF clear

EDX:EAX = current file position

if failed:

CF set

EAX = DOS error code

3.21 - DOS function 43h - Change File Attributes

==================================================

In:	AH = 43h

AL = function code

ECX = desired file attributes

DS:EDX = pointer to ASCIIZ file name

Out:	if successful:

CF clear

ECX = current attributes

if failed:

CF set

EAX = DOS error code

3.22 - DOS function 47h - Get Directory Path

==============================================

In:	AH = 47h

DL = drive number

DS:ESI = pointer to buffer for path

Out:	if successful:

CF clear

buffer filled with the path name

if failed:

CF set

EAX = DOS error code

3.23 - DOS function 48h - Allocate DOS Memory

===============================================

In:	AH = 48h

EBX = paragraphs to allocate

Out:	if successful:

CF clear

EAX = selector of allocated memory block

if failed:

CF set

EAX = DOS error code

EBX = maximum paragraphs available

3.24 - DOS function 49h - Deallocate DOS Memory

=================================================

In:	AH = 49h

ES = selector of previously allocated memory block

Out:	if successful:

CF clear

ES = NULL selector (0000h)

if failed:

CF set

EAX = DOS error code

3.25 - DOS function 4Ah - Resize DOS Memory

=============================================

In:	AH = 4Ah

EBX = new memory block size in paragraphs

ES = selector of previously allocated memory block

Out:	if successful:

CF clear

if failed:

CF set

EAX = DOS error code

EBX = maximum paragraphs available for specific memory block

3.26 - DOS function 4Bh - Execute Program (sub-function 00h)

==============================================================

In:	AH = 4Bh

AL = 00h

DS:EDX = pointer to ASCIIZ path name

ES:EBX = pointer to parameter block

Out:	if successful:

CF clear

if failed:

CF set

EAX = DOS error code

Notes:	a) DOS/32 Advanced will copy the environment provided by the protected

mode program into conventional (DOS) memory where it can be reached by

real mode DOS.

b) This function will return an error (EAX=0FFFFFFFFh and CF set) when

there was not enough conventional (DOS) memory for the protected mode

program's environment, or when the DPMI host could not allocate a

selector for the allocated memory block.

c) The only supported sub-function is AL=00h.

3.27 - DOS function 4Ch - Terminate Program

=============================================

In:	AH = 4Ch

AL = exit code

Out:	-

Notes:	a) This function never returns.

b) This function is callbacked by DOS/32 Advanced DPMI from real mode

to protected mode so that a protected mode clean up routine could

deallocate any extended memory and callbacks that was allocated at

DOS Extender's startup.

3.28 - DOS function 4Eh - Find First File

===========================================

In:	AH = 4Eh

ECX = file attributes

DS:EDX = pointer to ASCIIZ file name

Out:	if successful:

CF clear

DTA is filled with file information

if failed:

CF set

EAX = DOS error code

3.29 - DOS function 4Fh - Find Next File

==========================================

In:	AH = 4Fh

Out:	if successful:

CF clear

DTA is filled with file information

if failed:

CF set

EAX = DOS error code

3.30 - DOS function 51h - Get PSP Segment

===========================================

In:	AH = 51h

Out:	EBX = PSP segment

3.31 - DOS function 56h - Rename File

=======================================

In:	AH = 56h

DS:EDX = pointer to ASCIIZ old file name

ES:EDI = pointer to ASCIIZ new file name

Out:	if successful:

CF clear

if failed:

CF set

EAX = DOS error code

3.32 - DOS function 5Ah - Create Temp File

============================================

In:	AH = 5Ah

DS:EDX = pointer to ASCIIZ file name

Out:	if successful:

CF clear

EAX = handle

if failed:

CF set

EAX = DOS error code

3.33 - DOS function 5Bh - Create New File

===========================================

In:	AH = 5Bh

ECX = file attributes

DS:EDX = pointer to ASCIIZ file name

Out:	if successful:

CF clear

EAX = handle

if failed:

CF set

EAX = DOS error code

3.34 - DOS function 62h - Get PSP Selector

============================================

In:	AH = 62h

Out:	EBX = PSP selector

3.35 - DOS function 0FF00h - DOS/4G Identification call

=========================================================

In:	AX = 0FF00h

DX = 0078h

Out:	EAX = 0FFFF3447h ('..4G')

Notes:	a) This call is specific for DOS/4G(W) and DOS/32 Advanced

DOS Extenders only, and is not supported by standard DOS.

3.36 - DOS function 0FF88h - DOS/32 Advanced Identification call

==================================================================

In:	AX = 0FF88h

Out:	EAX = 49443332h ('ID32')

EBX = DOS/32 Advanced version number

ECX = DOS/32 Advanced internal data

EDX = DOS/32 Advanced internal data

ESI = DOS/32 Advanced internal data

EDI = DOS/32 Advanced internal data

Notes:	a) This call is specific for DOS/32 Advanced DOS Extender only, and

is not supported by standard DOS.

b) All general registers, except EBP, will be modified by this call.

Register EAX will contain the identification string, 'ID32', and

register EBX will contain the version number of currently running

DOS/32 Advanced DOS Extender. Note, that when spawning a program,

it is possible, using this call, to determine if the DOS/32 Advanced

DOS Extender is	running, and if that is the case, what version

of DOS/32 Advanced is in place.

c) This call is supported by DOS/32 Advanced in both real and

protected modes.

d) This call will be used by DOS/32 Advanced at startup, to determine

if the program was spawned from another DOS/32 Advanced application,

and to get information about extended memory that can be shared

between the spawned, and the host programs.

e) When an external DPMI host is present and runs the protected mode

(for exemple under Windows), this call will be supported by the DOS

Extender in protected mode only.

4.0 - Extended VGA/VESA BIOS functions - Overview

***************************************************

The following VGA/VESA functions listed below have been extended and are

available through software interrupt 10h (INT 10h) from protected mode. There

is no need to use DPMI function 0300h no more. Simply place the desired values

in specified registers and issue an INT 10h from your protected mode program.

Note, that extended VGA/VESA functions will not return carry flag set if the

function was unsuccessful. In addition, the high word of any 32bit register,

in which the function should return a 16bit value will be zeroed.

4.01 - VGA BIOS function 1Bh - Read Functionality Information

===============================================================

In:	AH = 1Bh

EBX = 0000h (implementation type)

ES:EDI = pointer to 64 byte buffer

Out:	AL = 1Bh (if function supported by VGA BIOS)

EBX = implementation type

ES:EDI = pointer to buffer filled with VGA BIOS information

Notes:	a) This call can be used to determine whether a VGA BIOS is supported

by the Video BIOS in ROM. If function returns AL=1Bh, the machine

is VGA compatible. To determine the presence of the VGA BIOS you can

also call VGA BIOS functions AX=1A00h and AH=1Ch.

4.02 - VGA BIOS function 1Ch - Save/Restore VGA State

=======================================================

In:	AH = 1Ch

ECX = states to save/restore:

bit 0 - VGA video hardware state

bit 1 - VGA BIOS data state

bit 2 - VGA DAC state

AL = subfunction:

00h - get state buffer size

Returns: EBX = number of 64-byte blocks needed

01h - save video states

02h - restore video states

ES:EBX = pointer to save/restore buffer

Out:	AL = 1Ch (if function supported by VGA BIOS)

Notes:	a) many BIOSes corrupt the video registers when saving the state,

so a program should restore the state immediately after saving it

(the saved data is uncorrupted).

4.03 - VESA BIOS function 4F00h - Get SuperVGA Information

============================================================

In:	AX = 4F00h

ES:EDI = pointer to 512 byte buffer

Out:	AL = 4Fh (if function supported by VESA BIOS)

AH = 00h (if function was successful)

ES:EDI = pointer to 512 byte buffer filled with VESA BIOS information

4.04 - VESA BIOS function 4F01h - Get SuperVGA Mode Information

=================================================================

In:	AX = 4F01h

ES:EDI = pointer to 256 byte buffer

Out:	AL = 4Fh (if function supported by VESA BIOS)

AH = 00h (if function was successful)

ES:EDI = pointer to 256 byte buffer filled with VESA mode information

4.05 - VESA BIOS function 4F04h - Save/Restore SuperVGA State

===============================================================

In:	AX = 4F04h

ECX = states to save/restore:

bit 0 - SVGA hardware state

bit 1 - SVGA BIOS data state

bit 2 - SVGA DAC state

bit 3 - SVGA state

DL = subfunction:

00h - get state buffer size

Returns: EBX = number of 64-byte blocks needed

01h - save video states

02h - restore video states

ES:EBX = pointer to save/restore buffer

Out:	AL = 4Fh (if function supported by VESA BIOS)

AH = 00h (if function was successful)

5.0 - Extended Mouse functions - Overview

*******************************************

Mouse functions that have been extended and listed below are available

through software interrupt 33h (INT 33h) in protected mode. Most of other

mouse functions (such as 0000h, 0001h, 0002h and so on) will also work when

called by your program from protected mode. The function number is placed in

register AX. Note that extended mouse functions do not return error codes.

Also note that if the DOS Extender at startup could not detect a mouse driver,

these functions will not be available from protected mode (the INT 33h

interrupt handler will simply return with an IRETD instruction).

5.01 - Mouse function 0009h - Define Graphics Cursor

======================================================

In:	AX = 0009h

EBX = column of cursor hot spot in bitmap

ECX = row of cursor hot spot in bitmap

ES:EDX = pointer to bitmap mask

Out:	-

5.02 - Mouse function 000Ch - Define Interrupt Subroutine Parameters

======================================================================

In:	AX = 000Ch

ECX = call mask

ES:EDX = far pointer to interrupt routine

Out:	-

Notes:	a) This function will use a real mode callback that was allocated by

the DOS/32 Advanced at startup to pass the control from real mode

mouse handler to your protected mode interrupt handler.

b) Calling this function with zero values in ES:EDX (0000:00000000h)

will uninstall your protected mode interrupt handler.

5.03 - Mouse function 0014h - Exchange Interrupt Subroutines

==============================================================

In:	AX = 0014h

ECX = call mask

ES:EDX = far pointer to interrupt routine

Out:	ECX = call mask of previous interrupt routine

ES:EDX = far pointer of previous interrupt routine

Notes:	a) This function will use a real mode callback that was allocated by

the DOS/32 Advanced at startup to pass the control from real mode

mouse handler to your protected mode interrupt handler.

b) Calling this function with zero values in ES:EDX (0000:00000000h)

will uninstall your protected mode interrupt handler.

5.04 - Mouse function 0016h - Save Driver State

=================================================

In:	AX = 0016h

EBX = size of buffer

ES:EDX = pointer to buffer to save informaion into

Out:	-

5.05 - Mouse function 0017h - Restore Driver State

====================================================

In:	AX = 0017h

EBX = size of buffer

ES:EDX = pointer to buffer containing saved information

Out:	-

5.06 - Mouse function 0018h - Set Alternate Mouse User Handler

================================================================

In:	AX = 0018h

ECX = call mask

ES:EDX = far pointer to routine to be invoked on mouse events

Out:	AX = 0018h on success

AX = 0FFFFh on error

Notes:	a) This function will use a real mode callback that was allocated by

the DOS/32 Advanced at startup to pass the control from real mode

mouse handler to your protected mode interrupt handler.

b) Calling this function with zero values in ES:EDX (0000:00000000h)

will uninstall your protected mode interrupt handler.

c) The protected mode program should not rely on error codes reported

by this function in AX register since different mouse drivers will

return different values on error.

5.07 - Mouse function 0019h - Return User Alternate Interrupt Handler

=======================================================================

In:	AX = 0019h

ECX = call mask

Out:	ECX = call mask

BX:EDX = user interrupt vector

Notes:	a) This function will simply return the address of the routine defined

by functions 000Ch, 0014h and 0018h.

5.08 - Mouse function 0020h - Enable Mouse Driver

===================================================

In:	AX = 0020h

Out:	AX = 0020h

AX = 0FFFFh

Notes:	a) This function will always return AX=0FFFFh to be compatible with

DOS/4GW DOS Extender.

b) The protected mode program should not rely on error codes reported

by this function in AX register since different mouse drivers will

return different values on error.

[END OF FILE]