Bells, Whistles, and Sound Boards

SOUND BLASTER FAMILY TECHNICAL REFERENCE

Copyright (c) 1993-97, Edward Schlunder

See COPYING.ES for redistribution details.

Contents:

1.      Introduction

1.1     I/O Address Map

1.2     DSP Commands

1.3     Mixer Registers

Part 1. Introduction

����������������������������������������������������������������������������

This information is from personal experimentation, information

gathered by other people, and by examining other people's source

code. It was gathered from many sources and therefore may contain

errors or omissions. I do not claim that any of this is "official"

in any way. If you find anything incorrect, please help others by

emailing to correction(s) to zilym@hndymn.stat.com.

1.1 I/O Address Map

Possible Base I/O: 210h, 220h, 230h, 240h (SB 1.xx)

220h, 240h             (All others)

Possible IRQ:      2, 3, 5, 7             (SB 1.xx)

2, 5, 7, 10            (All others)

Possible DMA:      1                      (SB 1.xx)

0, 1, 3                (SB 2.xx/SB Pro)

-����������������������������������������������������������������������-

� I/O Address �  Description/Usage                   � Read/Write    �

-����������������������������������������������������������������������-

Base + 0h      Left FM music Status Port            � Read       �

Base + 0h      Left FM music Register Address Port  � Write      �

Base + 1h      Left FM music Data Port              � Write      �

Base + 2h      Right FM music Status Port           � Read       �

Base + 2h      Right FM music Register Address Port � Write      �

Base + 3h      Right FM music Data Port             � Write      �

Base + 4h      Mixer chip Register Address Port     � Write      �

7 6 5 4 3 2 1 0

�������������Ĵ

Register Number �

See 'Mixer Registers' for details on all Mixer

Registers.

Base + 5h      Mixer chip Data Port                 � Read/Write �

After writing the Mixer register to read/write

on the 'Mixer chip Register Address Port', read or

write that register on this port. Data returned

varies on the register. See 'Mixer Registers' for

details.

Base + 6h      DSP Reset                            � Write      �

This is how you reset the DSP:

1. Send 1 to this port

2. Wait 3.3 microseconds

3. Send 0 to this port

4. Read from this port

5. If value read equals 0AAh, DSP was reset correctly

6. If value isn't 0AAh go back to step 4

7. If you have looped through steps 4-6 over 32

times and you haven't found 0AAh, then the DSP

wasn't correctly reset.

Base + 8h      FM music Status Port                 � Read       �

Base + 8h      FM music Register Address Port       � Write      �

Base + 9h      FM music Data Port                   � Write      �

Base + 0Ah     DSP Read Data                        � Read       �

Base + 0Ch     DSP Write Data or Command            � Write      �

7 6 5 4 3 2 1 0

�������������Ĵ

Data or Register �

See 'DSP Registers' for details on all DSP registers.

Base + 0Ch     DSP Write Buffer Status              � Read       �

7 6 5 4 3 2 1 0

� * * * * * * *

Available Status Bit �

Available Status:

0 -   Ready for data at Base + 0Ch

1 -   Not ready for data

Base + 0Eh     DSP Data Available Status            � Read       �

7 6 5 4 3 2 1 0

� * * * * * * *

Available Status Bit �

Available Status:

0 -   No data available at Base + 0Ah

1 -   Data available at Base + 0Ah

Base + 0Eh     8 Bit DMA IRQ Acknowledge port       � Read       �

Read this port after every 8 Bit DMA

transferred sound. This is usually done by the

IRQ handler set up before the transfer.

Base + 0Fh     16 Bit DMA IRQ Acknowledge port      � Read       �

Read this port after every 16 Bit DMA

transferred sound. This is usually done by the

IRQ handler set up before the transfer.

388h           FM music Status Port                 � Read       �

388h           FM music Register Address Port       � Write      �

389h           FM music Data Port                   � Write      �

1.2 DSP Commands

S-      Send

1. Read from Base+0Ch

2. Loop to 1 until Bit7 is clear

3. Write to Base+0Ch

R-      Read

1. Read from Base+0Eh

2. Loop to 1 until Bit7 is set

3. Read from Base+0Ah

NOT(x)- Logical NOT, (NOT x)

ex:   x=F1F2h. NOT x=0E0Dh

LSB(x)- Least Significant Byte (x MOD 256)

ex:   x=F1F2h. LSB(x)=F2h

MSB(x)- Most Significant Byte (x \ 256)

ex:   x=F1F2h. MSB(x)=F1h

CMD  VERSION(S)     PURPOSE/USAGE

���������������������������������������������������������������������������

010h 1.x-4.x        Play one Sample

S:10h, S:Sample

014h 1.x-4.x        Start Low Speed DMA Playback Transfer (8 Bit)

S:14h, S:LSB(Length-1), S:MSB(Length-1)

017h 1.x-4.x        Start Low Speed DMA Playback Transfer (2 Bit ADPCM)

S:17h, S:LSB(Length-1), S:MSB(Length-1)

01Ch 2.x-4.x        Start Low Speed Autoinit DMA Playback Transfer (8 Bit)

S:1Ch

020h 1.x-4.x        Record One Sample

S:20h, R:Sample

024h 1.x-4.x        Start Low Speed DMA Recording Transfer (8 Bit)

S:24h, S:LSB(Length-1), S:MSB(Length-1)

040h 1.x-4.x        DMA Set Sample Rate

S:40h, S:Sample Divisor

Sample Divisor = 256 - (1,000,000 \ Sample Rate)

041h 4.x            DMA Set Output Sample Rate (For commands Bxh & Cxh)

S:41h, S:MSB(Sample Rate), S:LSB(Sample Rate)

042h 4.x            DMA Set Input Sample Rate (For commands Bxh & Cxh)

S:42h, S:MSB(Sample Rate), S:LSB(Sample Rate)

048h 2.x-4.x        DMA Set Transfer Length (Commands 1Ch, 91h, 99h, 90h)

S:48h, S:LSB(Length-1), S:MSB(Length-1)

074h 1.x-4.x        Start Low Speed DMA Playback Transfer (4 Bit ADPCM)

S:74h, S:LSB(Length-1), S:MSB(Length-1)

077h 1.x-4.x        Start Low Speed DMA Playback Transfer (2.6 Bit ADPCM)

S:77h, S:LSB(Length-1), S:MSB(Length-1)

080h 1.x-4.x        Output Silent Block

S:80h, S:LSB(Length-1), S:MSB(Length-1)

090h 3.x            Start High Speed Autoinit DMA Playback Transfer (8 Bit)

S:90h

091h 2.x-4.x        Start High Speed DMA Playback Transfer (8 Bit)

S:91h

099h 2.x-4.x        Start High Speed DMA Recording Transfer (8 Bit)

S:99h

0Byh 4.x            Start 16Bit DMA Transfer

S:Byh, S:Mode, S:LSB(Length-1), S:MSB(Length-1)

Starts a 16Bit input/output DMA transfer. Note that

part of the command is used for data (y)..

Command equals:   7 6 5 4 3 2 1 0

Keep 1 � � � � � � � � Keep 0

Keep 0 � � � � � � 0=No FIFO, 1=16 Byte FIFO

Keep 1 � � � � 0=Single Cycle, 1=Autoinit

Keep 1 � � 0=Play 1=Record

Mode equals:   7 6 5 4 3 2 1 0

Keep 0 ��� � � ������� Keep 0

0=Mono 1=Stereo � �

0=Unsigned 1=Signed �

0Cyh 4.x            Start 8Bit DMA Transfer

S:Cyh, S:Mode, S:LSB(Length-1), S:MSB(Length-1)

Starts an 8Bit input/output DMA transfer.  Note that

part of the command is used for data (y)..

Command equals:   7 6 5 4 3 2 1 0

Keep 1 � � � � � � � � Keep 0

Keep 0 � � � � � � 0=No FIFO, 1=16 Byte FIFO

Keep 1 � � � � 0=Single Cycle, 1=Autoinit

Keep 1 � � 0=Play 1=Record

Mode equals:   7 6 5 4 3 2 1 0

Keep 0 ��� � � ������� Keep 0

0=Mono 1=Stereo � �

0=Unsigned 1=Signed �

0D0h 1.x-4.x        Pause 8Bit DMA Transfer

S:D0h

0D1h 1.x-4.x        Turn on DAC Output (no effect on DSP v4.x)

S:D1h

0D3h 1.x-4.x        Turn off DAC Output (no effect on DSP v4.x)

S:D3h

0D4h 1.x-4.x        Continue Paused 8Bit DMA Transfer

S:D0h

0D5h 4.x            Pause 16Bit DMA Transfer

S:D5h

0D6h 4.x            Continue Paused 16Bit DMA Transfer

S:D6h

0D8h 1.x-4.x        Read DAC Output Status

S:0D8h, R:Output Status (00=Disabled, FF=Enabled)

0D9h 4.x            Exit Autoinit Mode (16Bit)

S:D9h

0DAh 4.x            Exit Autoinit Mode (8Bit)

S:DAh

0E0h 1.x-4.x        Get DSP ID

S:E0h, S:tstval, R:NOT(tstval)

0E1h 1.x-4.x        Get DSP Version Number

S:E1h, R:Major Version, R:Minor Version

Ver:   Card Name:      Feature Addition:

1.xx   SB 1.xx         + low speed DMA transfers

2.xx   SB 2.xx         + high speed DMA transfers

+ DMA autoinitialize mode

3.00   SB Pro original + stereo FM/digital output

3.01   (same)

3.02   SB Pro-2        + OPL3 FM

4.xx   SB16            + 16Bit output

+ High Speed stereo DMA transfers

0E3h 1.x-4.x        Get Card Copyright String

??

0E4h 1.x-4.x        Get DSP ID #2

S:E4h, S:AAh, S:E8h, R:AAh

0F2h 1.x-2.x        Raise IRQ Line

S:F2h, (IRQ Occurs)

1.3 Mixer Registers

REGISTER NO.      PURPOSE/USAGE                      VERSION(S)

���������������������������������������������������������������������������

00h               Reset Mixer                        3.x-4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Any value �

Just send the Mixer 0 then an other number (any

will do) to reset it to the power on state.

04h               Voice Volume                       3.x-4.x

7 6 5 4 3 2 1 0

�����Ĵ �������

Left � � Right

0Ah               Microphone Volume                  3.x-4.x

7 6 5 4 3 2 1 0

* * * * * �����

� Recording Volume

0Ch               Input Filter                       3.x-4.x

7 6 5 4 3 2 1 0

* * � * � ��� *

Filter Enable �   � � Input Source

� Filter Select

Filter Select:          Input Source:

0 -    Low Frequency    00 -   Microphone

1 -    High Frequency   01 -   CD

11 -   Line-in

0Eh               Output Filter                      3.x-4.x

7 6 5 4 3 2 1 0

* * � * * * � *

Filter Enable �       � Stereo Enable

Filter Enable:

0 -    DNFI Filter enabled

1 -    DNFI Filter disabled

Stereo Enable:

0 -    Digital Sound output is Mono

1 -    Digital Sound output is Stereo (DMA mode only)

022h              Master Volume                      3.x-4.x

7 6 5 4 3 2 1 0

�����Ĵ �������

Left � � Right

026h              FM Music Volume                    3.x-4.x

7 6 5 4 3 2 1 0

�����Ĵ �������

Left � � Right

028h              CD Volume                          3.x-4.x

7 6 5 4 3 2 1 0

�����Ĵ �������

Left � � Right

02Eh              Line-in Volume                     3.x-4.x

7 6 5 4 3 2 1 0

�����Ĵ �������

Left � � Right

030h              Master Volume Left                 4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Left �

031h              Master Volume Left                 4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Right �

032h              Voice Volume Left                  4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Left �

033h              Voice Volume Right                 4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Right �

034h              FM Volume Left                     4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Left �

035h              FM Volume Right                    4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Right �

036h              CD Volume Left                     4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Left �

037h              CD Volume Right                    4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Right �

038h              Line-in Volume Left                4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Left �

039h              Line-in Volume Right               4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Right �

03Ah              Microphone Volume                  4.x

7 6 5 4 3 2 1 0

�������������Ĵ

Mic Vol �

03Bh              ???                                4.x

03Dh              ??? Left Volume                    4.x

03Eh              ??? Right Volume                   4.x